sixth edition

Fundamentals of

Electric Circuits

Charles K. Alexander

Department of Electrical and

Computer Engineering

Cleveland State University

Matthew N. O. Sadiku

Department of Electrical and

Computer Engineering

Prairie View A&M University

FUNDAMENTALS OF ELECTRIC CIRCUITS, SIXTH EDITION

Published by McGraw-Hill Education, 2 Penn Plaza, New York, NY 10121. Copyright © 2017 by  McGraw-Hill Education. All rights reserved. Printed in the United States of America. Previous editions  © 2013, 2009, and 2007. No part of this publication may be reproduced or distributed in any form or by  any means, or stored in a database or retrieval system, without the prior written consent of McGraw-Hill  Education, including, but not limited to, in any network or other electronic storage or transmission, or  broadcast for distance learning.

Some ancillaries, including electronic and print components, may not be available to customers outside  the United States.

This book is printed on acid-free paper.

1 2 3 4 5 6 7 8 9 0 DOW/DOW 1 0 9 8 7 6 5

ISBN 978-0-07-802822-9

MHID 0-07-802822-1

Senior Vice President, Products & Markets: Kurt L. Strand

Vice President, General Manager, Products & Markets: Marty Lange

Vice President, Content Design & Delivery: Kimberly Meriwether David

Managing Director: Thomas Timp

Global Brand Manager: Raghu Srinivasan

Director, Product Development: Rose Koos

Product Developer: Vincent Bradshaw

Marketing Manager: Nick McFadden

Digital Product Analyst: Patrick Diller

Associate Director of Digital Content: Amy Bumbaco, Ph.D.

Director, Content Design & Delivery: Linda Avenarius

Program Manager: Faye M. Herrig

Content Project Managers: Melissa M. Leick, Tammy Juran, Sandra Schnee

Buyer: Sandy Ludovissy

Design: Studio Montage, Inc.

Content Licensing Specialist: Lorraine Buczek

Cover Image: Courtesy NASA/JPL - Caltech

Compositor: MPS Limited

Printer: R. R. Donnelley

All credits appearing on page or at the end of the book are considered to be an extension of the  copyright page.

Library of Congress Cataloging-in-Publication Data

Alexander, Charles K., author.

Fundamentals of electric circuits / Charles K. Alexander, Department of Electrical and Computer  Engineering, Cleveland State University, Matthew N. O. Sadiku, Department of Electrical Engineering,  Prairie View A&M University. — Sixth edition.

pages cm

Includes index.

ISBN 978-0-07-802822-9 (alk. paper) — ISBN 0-07-802822-1 (alk. paper)

1. Electric circuits. I. Sadiku, Matthew N. O., author. II. Title.

TK454.A452 2017

621.3815—dc23 2015035301

The Internet addresses listed in the text were accurate at the time of publication. The inclusion of a  website does not indicate an endorsement by the authors or McGraw-Hill Education, and McGraw-Hill  Education does not guarantee the accuracy of the information presented at these sites.

mheducation.com/highered

Dedicated to our wives, Kikelomo and Hannah, whose understanding and   support have truly made this book possible.

Matthew

and

Chuck

Contents

Preface xi

Acknowledgments xv

About the Authors xxi

PART 1 DC Circuits 2

Chapter 1 Basic Concepts 3

1.1 Introduction 4

1.2 Systems of Units 5

1.3 Charge and Current 6

1.4 Voltage 9

1.5 Power and Energy 10

1.6 Circuit Elements 14

1.7 Applications 16

1.7.1 TV Picture Tube

1.7.2 Electricity Bills

1.8 Problem Solving 19

1.9 Summary 22

Review Questions 23

Problems 24

Comprehensive Problems 26

Chapter 2 Basic Laws 29

2.1 Introduction 30

2.2 Ohm’s Law 30

2.3 Nodes, Branches, and Loops 35 2.4 Kirchhoff’s Laws 37

2.5 Series Resistors and Voltage Division 43 2.6 Parallel Resistors and Current Division 44 2.7 Wye-Delta Transformations 51

Delta to Wye Conversion

Wye to Delta Conversion

2.8 Applications 57

2.8.1 Lighting Systems

2.8.2 Design of DC Meters

2.9 Summary 63

Review Questions 64

Problems 65

Comprehensive Problems 77

Chapter 3 Methods of Analysis 79

3.1 Introduction 80

3.2 Nodal Analysis 80

3.3 Nodal Analysis with Voltage Sources 86 3.4 Mesh Analysis 91

3.5 Mesh Analysis with Current Sources 96 3.6 Nodal and Mesh Analyses

by Inspection 98

3.7 Nodal Versus Mesh Analysis 102 3.8 Circuit Analysis with PSpice 103 3.9 Applications: DC Transistor Circuits 105 3.10 Summary 110

Review Questions 111

Problems 112

Comprehensive Problem 124

Chapter 4 Circuit Theorems 125

4.1 Introduction 126

4.2 Linearity Property 126

4.3 Superposition 128

4.4 Source Transformation 133

4.5 Thevenin’s Theorem 137

4.6 Norton’s Theorem 143

4.7 Derivations of Thevenin’s

and Norton’s Theorems 147

4.8 Maximum Power Transfer 148 4.9 Verifying Circuit Theorems

with PSpice 150

4.10 Applications 153

4.10.1 Source Modeling

4.10.2 Resistance Measurement

4.11 Summary 158

Review Questions 159

Problems 160

Comprehensive Problems 171

Chapter 5 Operational Amplifiers 173

5.1 Introduction 174

5.2 Operational Amplifiers 174

v

vi Contents

5.3 Ideal Op Amp 178

5.4 Inverting Amplifier 179

5.5 Noninverting Amplifier 181

5.6 Summing Amplifier 183

5.7 Difference Amplifier 185

5.8 Cascaded Op Amp Circuits 189 5.9 Op Amp Circuit Analysis with PSpice 192 5.10 Applications 194

5.10.1 Digital-to-Analog Converter

5.10.2 Instrumentation Amplifiers

5.11 Summary 197

Review Questions 199

Problems 200

Comprehensive Problems 211

Chapter 6 Capacitors and Inductors 213

6.1 Introduction 214

6.2 Capacitors 214

6.3 Series and Parallel Capacitors 220 6.4 Inductors 224

6.5 Series and Parallel Inductors 228 6.6 Applications 231

6.6.1 Integrator

6.6.2 Differentiator

6.6.3 Analog Computer

6.7 Summary 238

Review Questions 239

Problems 240

Comprehensive Problems 249

Chapter 7 First-Order Circuits  251

7.1 Introduction 252

7.2 The Source-Free RC Circuit 253

7.3 The Source-Free RL Circuit 257

7.4 Singularity Functions 263

7.5 Step Response of an RC Circuit 271 7.6 Step Response of an RL Circuit 278 7.7 First-Order Op Amp Circuits 282 7.8 Transient Analysis with PSpice 287 7.9 Applications 291

7.9.1 Delay Circuits

7.9.2 Photoflash Unit

7.9.3 Relay Circuits

7.9.4 Automobile Ignition Circuit

7.10 Summary 297

Review Questions 298

Problems 299

Comprehensive Problems 309

Chapter 8 Second-Order Circuits  311

8.1 Introduction 312

8.2 Finding Initial and Final Values 313 8.3 The Source-Free Series

RLC Circuit 317

8.4 The Source-Free Parallel

RLC Circuit 324

8.5 Step Response of a Series RLC  Circuit 329

8.6 Step Response of a Parallel RLC  Circuit 334

8.7 General Second-Order Circuits 337 8.8 Second-Order Op Amp Circuits 342 8.9 PSpice Analysis of RLC Circuits 344 8.10 Duality 348

8.11 Applications 351

8.11.1 Automobile Ignition System

8.11.2 Smoothing Circuits

8.12 Summary 354

Review Questions 355

Problems 356

Comprehensive Problems 365

PART 2 AC Circuits  366

Chapter 9 Sinusoids and Phasors 367

9.1 Introduction 368

9.2 Sinusoids 369

9.3 Phasors 374

9.4 Phasor Relationships for

Circuit Elements 383

9.5 Impedance and Admittance 385 9.6 Kirchhoff’s Laws in the Frequency  Domain 387

9.7 Impedance Combinations 388 9.8 Applications 394

9.8.1 Phase-Shifters

9.8.2 AC Bridges

9.9 Summary 400

Review Questions 401

Problems 401

Comprehensive Problems 409

Chapter 10 Sinusoidal Steady-State  Analysis 411

10.1 Introduction 412

10.2 Nodal Analysis 412

10.3 Mesh Analysis 415

10.4 Superposition Theorem 419 10.5 Source Transformation 422 10.6 Thevenin and Norton

Equivalent Circuits 424

10.7 Op Amp AC Circuits 429

10.8 AC Analysis Using PSpice 431 10.9 Applications 435

10.9.1 Capacitance Multiplier

10.9.2 Oscillators

10.10 Summary 439

Review Questions 439

Problems 441

Chapter 11 AC Power Analysis 455

11.1 Introduction 456

11.2 Instantaneous and Average Power 456 11.3 Maximum Average Power Transfer 462 11.4 Effective or RMS Value 465 11.5 Apparent Power and

Power Factor 468

11.6 Complex Power 471

11.7 Conservation of AC Power 475 11.8 Power Factor Correction 479 11.9 Applications 481

11.9.1 Power Measurement

11.9.2 Electricity Consumption Cost

11.10 Summary 486

Review Questions 488

Problems 488

Comprehensive Problems 498

Chapter 12 Three-Phase Circuits 501

12.1 Introduction 502

12.2 Balanced Three-Phase Voltages 503 12.3 Balanced Wye-Wye Connection 507 12.4 Balanced Wye-Delta Connection 510 12.5 Balanced Delta-Delta

Connection 512

12.6 Balanced Delta-Wye Connection 514 12.7 Power in a Balanced System 517 12.8 Unbalanced Three-Phase

Systems 523

12.9 PSpice for Three-Phase Circuits 527 12.10 Applications 532

12.10.1 Three-Phase Power Measurement 12.10.2 Residential Wiring

Contents vii

12.11 Summary 541

Review Questions 541

Problems 542

Comprehensive Problems 551

Chapter 13 Magnetically Coupled  Circuits 553

13.1 Introduction 554

13.2 Mutual Inductance 555

13.3 Energy in a Coupled Circuit 562 13.4 Linear Transformers 565

13.5 Ideal Transformers 571

13.6 Ideal Autotransformers 579

13.7 Three-Phase Transformers 582 13.8 PSpice Analysis of Magnetically  Coupled Circuits 584

13.9 Applications 589

13.9.1 Transformer as an Isolation Device 13.9.2 Transformer as a Matching Device 13.9.3 Power Distribution

13.10 Summary 595

Review Questions 596

Problems 597

Comprehensive Problems 609

Chapter 14 Frequency Response 611

14.1 Introduction 612

14.2 Transfer Function 612

14.3 The Decibel Scale 615

14.4 Bode Plots 617

14.5 Series Resonance 627

14.6 Parallel Resonance 632

14.7 Passive Filters 635

14.7.1 Low-Pass Filter

14.7.2 High-Pass Filter

14.7.3 Band-Pass Filter

14.7.4 Band-Stop Filter

14.8 Active Filters 640

14.8.1 First-Order Low-Pass Filter

14.8.2 First-Order High-Pass Filter

14.8.3 Band-Pass Filter

14.8.4 Band-Reject (or Notch) Filter

14.9 Scaling 646

14.9.1 Magnitude Scaling

14.9.2 Frequency Scaling

14.9.3 Magnitude and Frequency Scaling

viii Contents

14.10 Frequency Response Using

PSpice 650

14.11 Computation Using MATLAB 653 14.12 Applications 655

14.12.1 Radio Receiver

14.12.2 Touch-Tone Telephone

14.12.3 Crossover Network

14.13 Summary 661

Review Questions 662

Problems 663

Comprehensive Problems 671

PART 3 Advanced Circuit

Analysis 672

Chapter 15 Introduction to the Laplace  Transform 673

15.1 Introduction 674

15.2 Definition of the Laplace

Transform 675

15.3 Properties of the Laplace

Transform 677

15.4 The Inverse Laplace Transform 688 15.4.1 Simple Poles

15.4.2 Repeated Poles

15.4.3 Complex Poles

15.5 The Convolution Integral 695

15.6 Application to Integrodifferential   Equations 703

15.7 Summary 706

Review Questions 706

Problems 707

Chapter 16 Applications of the Laplace  Transform 713

16.1 Introduction 714

16.2 Circuit Element Models 715

16.3 Circuit Analysis 720

16.4 Transfer Functions 724

16.5 State Variables 728

16.6 Applications 735

16.6.1 Network Stability

16.6.2 Network Synthesis

16.7 Summary 743

Review Questions 744

Problems 745

Comprehensive Problems 756

Chapter 17 The Fourier Series 757

17.1 Introduction 758

17.2 Trigonometric Fourier Series 759 17.3 Symmetry Considerations 766 17.3.1 Even Symmetry

17.3.2 Odd Symmetry

17.3.3 Half-Wave Symmetry

17.4 Circuit Applications 776

17.5 Average Power and RMS Values 780 17.6 Exponential Fourier Series 783 17.7 Fourier Analysis with PSpice 789 17.7.1 Discrete Fourier Transform

17.7.2 Fast Fourier Transform

17.8 Applications 795

17.8.1 Spectrum Analyzers

17.8.2 Filters

17.9 Summary 798

Review Questions 800

Problems 800

Comprehensive Problems 809

Chapter 18 Fourier Transform 811

18.1 Introduction 812

18.2 Definition of the Fourier Transform 812 18.3 Properties of the Fourier

Transform 818

18.4 Circuit Applications 831

18.5 Parseval’s Theorem 834

18.6 Comparing the Fourier and

Laplace Transforms 837

18.7 Applications 838

18.7.1 Amplitude Modulation

18.7.2 Sampling

18.8 Summary 841

Review Questions 842

Problems 843

Comprehensive Problems 849

Chapter 19 Two-Port Networks 851

19.1 Introduction 852

19.2 Impedance Parameters 853 19.3 Admittance Parameters 857 19.4 Hybrid Parameters 860

19.5 Transmission Parameters 865 19.6 Relationships Between

Parameters 870

19.7 Interconnection of Networks 873 19.8 Computing Two-Port Parameters  Using PSpice 879

19.9 Applications 882

19.9.1 Transistor Circuits

19.9.2 Ladder Network Synthesis

19.10 Summary 891

Review Questions 892

Problems 892

Comprehensive Problem 903

Contents ix

Appendix A Simultaneous Equations and Matrix  Inversion A

Appendix B Complex Numbers A-9 Appendix C Mathematical Formulas A-16

Appendix D Answers to Odd-Numbered   Problems A-21

Selected Bibliography B-1

Index I-1

Preface

In keeping with our focus on space for covers for our book, we have  chosen the NASA Voyager spacecraft for the sixth edition. The reason  for this is that like any spacecraft there are many circuits that play criti - cal roles in their functionality. The beginning of the Voyager 1 and 2  odyssey began on August 20, 1977, for Voyager 2 and on September 5,  1977, for Voyager 1. Both were launched from NASA’s Kennedy Space  Center in Florida. The Voyager 1 was launched on a faster orbit so it  eventually became the first man-made object to leave our solar system.  There is some debate over whether it has actually left the solar system,  but it certainly will at some point in time. Voyager 2 and two Pioneer  spacecraft will also leave the solar system at some point in time.

Voyager 1 is still functioning and sending back data, a truly signifi cant achievement for NASA engineers. The design processes that make  the Voyager operate so reliably are based on the fundamentals discussed  in this textbook. Finally, space is vast so that Voyager 1 will fly past  other solar systems; the odds of actually coming into contact with some thing are so remote that it may virtually fly through the universe forever!  For more about Voyager 1, go to NASA’s website: www.nasa.gov/.

Features

New to This Edition

We have added learning objectives to each chapter to reflect what we believe are the most important items to learn from each chapter. These should help you focus more carefully on what you should be learning.

There are more than 580 revised end-of-chapter problems, new end of-chapter problems, and revised practice problems. We continue to try and  make our problems as practical as possible.

In addition, we have improved Connect for this edition by increasing  the number of problems available substantially. Now, professors may select  from more than a thousand problems as they build thier online homework  assignments.

We have also built SmartBook for this edition. With SmartBook, stu - dents get the same text as the print version, along with personalized tips on  what to study next, thanks to SmartBook’s adaptive technology.

Retained from Previous Editions

A course in circuit analysis is perhaps the first exposure students have to electrical engineering. This is also a place where we can enhance  some of the skills that they will later need as they learn how to design.  An important part of this book is our 121 design a problem problems.  These problems were developed to enhance skills that are an impor - tant part of the design process. We know it is not possible to fully de velop a student’s design skills in a fundamental course like circuits.  To fully develop design skills a student needs a design experience

xi

xii Preface

normally reserved for their senior year. This does not mean that some

of those skills cannot be developed and exercised in a circuits course.

The text already included open-ended questions that help students

use creativity, which is an important part of learning how to design.

We already have some questions that are open-ended but we desired

to add much more into our text in this important area and have devel-

oped an approach to do just that. When we develop problems for the

student to solve our goal is that in solving the problem the student

learns more about the theory and the problem solving process. Why

not have the students design problems like we do? That is exactly

what we do in each chapter. Within the normal problem set, we have

a set of problems where we ask the student to design a problem to

help other students better understand an important concept. This has

two very important results. The first will be a better understanding of

the basic theory and the second will be the enhancement of some of

the student’s basic design skills. We are making effective use of the

principle of learning by teaching. Essentially we all learn better when

we teach a subject. Designing effective problems is a key part of

the teaching process. Students should also be encouraged to develop

problems, when appropriate, which have nice numbers and do not

necessarily overemphasize complicated mathematical manipulations.

A very important advantage to our textbook, we have a total of 2,481

Examples, Practice Problems, Review Questions, and End-of-Chapter

Problems! Answers are provided for all practice problems and the odd

numbered end-of-chapter problems.

The main objective of the sixth edition of this book remains the

same as the previous editions—to present circuit analysis in a manner

that is clearer, more interesting, and easier to understand than other cir-

cuit textbooks, and to assist the student in beginning to see the “fun” in

engineering. This objective is achieved in the following ways:

• Chapter Openers and Summaries

Each chapter opens with a discussion about how to enhance skills

which contribute to successful problem solving as well as success-

ful careers or a career-oriented talk on a subdiscipline of electrical

engineering. This is followed by an introduction that links the chap-

ter with the previous chapters and states the chapter objectives. The

chapter ends with a summary of key points and formulas.

• Problem-Solving Methodology

Chapter 1 introduces a six-step method for solving circuit problems

which is used consistently throughout the book and media supple-

ments to promote best-practice problem-solving procedures.

• Student-Friendly Writing Style

All principles are presented in a lucid, logical, step-by-step man-

ner. As much as possible, we avoid wordiness and giving too much

detail that could hide concepts and impede overall understanding of

the material.

• Boxed Formulas and Key Terms

Important formulas are boxed as a means of helping students sort

out what is essential from what is not. Also, to ensure that students

clearly understand the key elements of the subject matter, key terms

are defined and highlighted.

Preface xiii

• Margin Notes

Marginal notes are used as a pedagogical aid. They serve multiple

uses such as hints, cross-references, more exposition, warnings,

reminders not to make some particular common mistakes, and prob-

lem-solving insights.

• Worked Examples

Thoroughly worked examples are liberally given at the end of ev-

ery section. The examples are regarded as a part of the text and are

clearly explained without asking the reader to fill in missing steps.

Thoroughly worked examples give students a good understanding

of the solution process and the confidence to solve problems them-

selves. Some of the problems are solved in two or three different

ways to facilitate a substantial comprehension of the subject mate-

rial as well as a comparison of different approaches.

• Practice Problems

To give students practice opportunity, each illustrative example is

immediately followed by a practice problem with the answer. The

student can follow the example step-by-step to aid in the solution of

the practice problem without flipping pages or looking at the end of

the book for answers. The practice problem is also intended to test a

student’s understanding of the preceding example. It will reinforce

their grasp of the material before the student can move on to the

next section. Complete solutions to the practice problems are avail-

able to students on the website.

• Application Sections

The last section in each chapter is devoted to practical application

aspects of the concepts covered in the chapter. The material covered

in the chapter is applied to at least one or two practical problems

or devices. This helps students see how the concepts are applied to

real-life situations.

• Review Questions

Ten review questions in the form of multiple-choice objective items

are provided at the end of each chapter with answers. The review

questions are intended to cover the little “tricks” that the examples

and end-of-chapter problems may not cover. They serve as a self

test device and help students determine how well they have mas-

tered the chapter.

• Computer Tools

In recognition of the requirements by ABET® on integrating

computer tools, the use of PSpice, Multisim, MATLAB, KCIDE for

Circuits, and developing design skills are encouraged in a student

friendly manner. PSpice is covered early on in the text so that stu-

dents can become familiar and use it throughout the text. Tutorials

on all of these are available on Connect. MATLAB is also introduced

early in the book.

• Design a Problem Problems

Finally, design a problem problems are meant to help the student de-

velop skills that will be needed in the design process.

• Historical Tidbits

Historical sketches throughout the text provide profiles of important

pioneers and events relevant to the study of electrical engineering.

xiv Preface

• Early Op Amp Discussion

The operational amplifier (op amp) as a basic element is introduced

early in the text.

• Fourier and Laplace Transforms Coverage

To ease the transition between the circuit course and signals and

systems courses, Fourier and Laplace transforms are covered lu-

cidly and thoroughly. The chapters are developed in a manner that

the interested instructor can go from solutions of first-order circuits

to Chapter 15. This then allows a very natural progression from

Laplace to Fourier to AC.

• Four-Color Art Program

An interior design and four-color art program bring circuit drawings

to life and enhance key pedagogical elements throughout the text.

• Extended Examples

Examples worked in detail according to the six-step problem solv-

ing method provide a road map for students to solve problems in a

consistent fashion. At least one example in each chapter is devel-

oped in this manner.

• EC 2000 Chapter Openers

Based on ABET’s skill-based CRITERION 3, these chapter openers

are devoted to discussions as to how students can acquire the skills

that will lead to a significantly enhanced career as an engineer. Be-

cause these skills are so very important to the student while still in

college as well after graduation, we use the heading, “Enhancing

your Skills and your Career.”

• Homework Problems

There are 580 new or revised end-of-chapter problems and changed

practice problems which will provide students with plenty of practice

as well as reinforce key concepts.

• Homework Problem Icons

Icons are used to highlight problems that relate to engineering de-

sign as well as problems that can be solved using PSpice, Multisim,

KCIDE, or MATLAB.

Organization

This book was written for a two-semester or three-quarter course in

linear circuit analysis. The book may also be used for a one-semester

course by a proper selection of chapters and sections by the instructor. It

is broadly divided into three parts.

• Part 1, consisting of Chapters 1 to 8, is devoted to dc circuits. It

covers the fundamental laws and theorems, circuits techniques, and

passive and active elements.

• Part 2, which contains Chapter 9 to 14, deals with ac circuits. It

introduces phasors, sinusoidal steady-state analysis, ac power, rms

values, three-phase systems, and frequency response.

• Part 3, consisting of Chapters 15 to 19, are devoted to advanced

techniques for network analysis. It provides students with a solid

introduction to the Laplace transform, Fourier series, Fourier trans-

form, and two-port network analysis.

Preface xv

The material in the three parts is more than sufficient for a two-semester

course, so the instructor must select which chapters or sections to cover.

Sections marked with the dagger sign (†) may be skipped, explained

briefly, or assigned as homework. They can be omitted without loss of

continuity. Each chapter has plenty of problems grouped according to the

sections of the related material and diverse enough that the instructor can

choose some as examples and assign some as homework. As stated ear-

lier, we are using three icons with this edition. We are using to de-

note problems that either require PSpice in the solution process, where

the circuit complexity is such that PSpice or Multisim would make the

solution process easier, and where PSpice or Multisim makes a good

check to see if the problem has been solved correctly. We are using

to denote problems where MATLAB is required in the solution process,

where MATLAB makes sense because of the problem makeup and its

complexity, and where MATLAB makes a good check to see if the prob

lem has been solved correctly. Finally, we use to identify problems

that help the student develop skills that are needed for engineering

design. More difficult problems are marked with an asterisk (*).

Comprehensive problems follow the end-of-chapter problems. They

are mostly applications problems that require skills learned from that

particular chapter.

Prerequisites

As with most introductory circuit courses, the main prerequisites, for a

course using this textbook, are physics and calculus. Although familiar-

ity with complex numbers is helpful in the later part of the book, it is not

required. A very important asset of this text is that ALL the mathemati-

cal equations and fundamentals of physics needed by the student, are

included in the text.

Acknowledgments

We would like to express our appreciation for the loving support we

have received from our wives (Hannah and Kikelomo), daughters

(Christina, Tamara, Jennifer, Motunrayo, Ann, and Joyce), son (Baixi),

and our extended family members. We sincerely appreciate the invalu

able help given us by Richard Rarick in helping us make the sixth edi

tion a significantly more relevant book. He has checked all the new and

revised problems and offered advice on making them more accurate

and clear.

At McGraw-Hill, we would like to thank the following editorial

and production staff: Raghu Srinivasan, global brand manager; Vincent

Bradshaw, product developer; Nick McFadden, marketing manager;

and Melissa Leick, content project manager.

The sixth edition has benefited greatly from the many outstanding

individuals who have offered suggestions for improvements in both the

text as well as the various problems. In particular, we thank Nicholas

Reeder, Professor of Electronics Engineering Technology, Sinclair

Community College, Dayton, Ohio, and Douglas De Boer, Professor

of Engineering, Dordt College, Sioux Center, Iowa, for their detailed

and careful corrections and suggestions for clarification which have

xvi Preface

contributed to making this a better edition. In addition, the follow -

ing have made important contributions to this edition (in alphabetical

order):

Zekeriya Aliyazicioglu, California State Polytechnic University—

Pomona

Rajan Chandra, California State Polytechnic University—Pomona

Mohammad Haider, University of Alabama—Birmingham

John Heathcote, Reedley College

Peter LoPresti, University of Tulsa

Robert Norwood, John Brown University

Aaron Ohta, University of Hawaii—Manoa

Salomon Oldak, California State Polytechnic University—Pomona

Hesham Shaalan, U.S. Merchant Marine Academy

Surendra Singh, University of Tulsa

Finally, we sincerely appreciate the feedback received from instructors

and students who used the previous editions. We want this to continue, so

please keep sending us e-mails or direct them to the publisher. We can be

reached at c.alexander@ieee.org for Charles Alexander and sadiku@ieee

.org for Matthew Sadiku.

C. K. Alexander and M. N. O. Sadiku

Supplements

Instructor and Student Resources

Available on Connect are a number of additional instructor and student

resources to accompany the text. These include complete solutions for

all practice and end-of-chapter problems, solutions in PSpice and Mul

tisim problems, lecture PowerPoints®, and text image files. In addition,

instructors can use COSMOS, a complete online solutions manual or

ganization system to create custom homework, quizzes, and tests using

end-of-chapter problems from the text.

Knowledge Capturing Integrated Design

Environment for Circuits (KCIDE for Circuits)

This software, developed at Cleveland State University and funded by

NASA, is designed to help the student work through a circuits problem

in an organized manner using the six-step problem-solving methodology

in the text. KCIDE for Circuits allows students to work a circuit problem

in PSpice and MATLAB, track the evolution of their solution, and save a

record of their process for future reference. In addition, the software auto

matically generates a Word document and/or a PowerPoint presentation.

The software package can be downloaded for free.

It is hoped that the book and supplemental materials supply the in -

structor with all the pedagogical tools necessary to effectively present

the material.

McGraw-Hill Create®

Craft your teaching resources to match the way you teach! With McGraw

Hill Create, http://create.mheducation.com, you can easily rearrange chap

ters, combine material from other content sources, and quickly upload

This page intentionally left blank

About the Authors

Charles K. Alexander is professor of electrical and computer engineer ing in the Washkewicz College of Engineering at Cleveland State Univer sity, Cleveland, Ohio. He is also the director of the Center for Research  in Electronics and Aerospace Technology (CREATE). From 2002 until

2006 he was dean of the Fenn College of Engineering. He has held the  position of dean of engineering at Cleveland State University, California  State University, Northridge, and Temple University (acting dean for six  years). He has held the position of department chair at Temple Univer sity and Tennessee Technological University as well as the position of  Stocker Visiting Professor (an endowed chair) at Ohio University. He  has held faculty status at all of the aforementioned universities.

Dr. Alexander has secured funding for two centers of research at  Ohio University and Cleveland State University. He has been the direc tor of three additional research centers at Temple and Tennessee Tech  and has obtained research funding of approximately $100 million (in  today’s dollars). He has served as a consultant to 23 private and govern mental organizations including the Air Force and the Navy.

He received the honorary Dr. Eng. from Ohio Northern University  (2009), his PhD (1971) and M.S.E.E. (1967) from Ohio University, and  the B.S.E.E. (1965) from Ohio Northern University.

Dr. Alexander has authored many publications, including a work - book and a videotape lecture series, and is coauthor of Fundamentals  of Electric Circuits, currently in its fifth edition, Engineering Skills for  Career Success, Problem Solving Made ALMOST Easy, the fifth edi tion of the Standard Handbook of Electronic Engineering, and Applied  Circuit Analysis, all with McGraw-Hill. He has delivered more than  500 paper, professional, and technical presentations.

Dr. Alexander is a Life Fellow of the IEEE and served as its presi dent and CEO in 1997. In addition he has held several volunteer posi - tions within the IEEE during his more than 45 years of service. This  includes serving from 1991 to 1999 on the IEEE board of directors.

He has received several local, regional, national, and international  awards for teaching and research, including an honorary Doctor of Engi neering degree, Fellow of the IEEE, the IEEE-USA Jim Watson Student  Professional Awareness Achievement Award, the IEEE Undergraduate  Teaching Award, the Distinguished Professor Award, the Distinguished  Engineering Education Achievement Award, the Distinguished Engi - neering Education Leadership Award, the IEEE Centennial Medal, and  the IEEE/RAB Innovation Award.

Charles K. Alexander

xxi

xxii About the Authors

Matthew N. O. Sadiku received his PhD from Tennessee Technological

University, Cookeville. From 1984 to 1988, he was an assistant professor

at Florida Atlantic University, where he did graduate work in computer

science. From 1988 to 2000, he was at Temple University, Philadelphia,

Pennsylvania, where he became a full professor. From 2000 to 2002,

he was with Lucent/Avaya, Holmdel, New Jersey, as a system engineer

and with Boeing Satellite Systems as a senior scientist. He is currently a

professor at Prairie View A&M University.

Dr. Sadiku is the author of more than 240 professional papers and

over 60 books, including Elements of Electromagnetics (Oxford Uni -

versity Press, 6th ed., 2015), Numerical Techniques in Electromagnet

ics with MATLAB (CRC, 3rd ed., 2009), and Metropolitan Area Net -

works (CRC Press, 1995). Some of his books have been translated into

French, Korean, Chinese (and Chinese Long Form in Taiwan), Italian,

Portuguese, and Spanish. He was the recipient of the 2000 McGraw-Hill/

Jacob Millman Award for outstanding contributions in the field of elec

trical engineering. He was also the recipient of Regents Professor award

Matthew N. O. Sadiku

for 2012 to 2013 by the Texas A&M University System. His current research interests are in the areas of numerical modeling  of electromagnetic systems and computer communication networks. He  is a registered professional engineer and a fellow of the Institute of Elec trical and Electronics Engineers (IEEE) “for contributions to computa - tional electromagnetics and engineering education.” He was the IEEE  Region 2 Student Activities Committee Chairman. He was an associ - ate editor for IEEE Transactions on Education and is a member of the   Association for Computing Machinery (ACM).

Fundamentals of

Electric Circuits

PART ONE

DC Ci rcuits

OUTLINE

1 Basic Concepts

2 Basic Laws

3 Methods of Analysis

4 Circuit Theorems

5 Operational Amplifiers

6 Capacitors and Inductors

7 First-Order Circuits

8 Second-Order Circuits

NASA

Basic Concepts

Some books are to be tasted, others to be swallowed, and some few to be  chewed and digested.

—Francis Bacon

Enhancing Your Skills and Your Career

ABET EC 2000 criteria (3.a), “an ability to apply knowledge  of mathematics, science, and engineering.”

As students, you are required to study mathematics, science, and engi neering with the purpose of being able to apply that kno wledge to the  solution of engineering problems. The skill here is the ability to apply  the fundamentals of these areas in the solution of a problem. So how do

you develop and enhance this skill?

The best approach is to w ork as man y problems as possible in all  of your courses. However, if you are really going to be successful with  this, you must spend time analyzing where and when and why you have  difficulty in easily arriving at successful solutions. You may be surprised  to learn that most of your problem-solving problems are with mathemat ics rather than your understanding of theory . You may also learn that  you start working the problem too soon. Taking time to think about the  problem and ho w you should solv e it will al ways save you time and  frustration in the end.

What I have found that works best for me is to apply our six step  problem-solving technique. Then I carefully identify the areas  where I ha ve dif ficulty solving the problem. Many times, my actual  deficiencies are in my understanding and ability to use correctly certain  mathematical principles. I then return to my fundamental math texts and  carefully review the appropriate sections, and in some cases, work some  example problems in that text. This brings me to another important thing  you should always do: Keep nearby all your basic mathematics, science,  and engineering textbooks.

This process of continually looking up material you thought you  had acquired in earlier courses may seem v ery tedious at first; however,  as your skills de velop and your kno wledge increases, this process will  become easier and easier. On a personal note, it is this very process that  led me from being a much less than a verage student to someone who  could earn a Ph.D. and become a successful researcher.

chapter 1

Photo by Charles Alexander

3

4 Chapter 1 Basic Concepts

Learning Objectives

By using the information and exercises in this chapter you will be

able to:

1. Understand the different units with which engineers work.

2. Understand the relationship between charge and current and

how to use both in a variety of applications.

3. Understand voltage and how it can be used in a variety of

applications.

4. Develop an understanding of power and energy and their

relationship with current and voltage.

5. Begin to understand the volt-amp characteristics of a variety of

circuit elements.

6. Begin to understand an organized approach to problem solving

and how it can be used to assist in your efforts to solve circuit

problems.

1.1 Introduction

Electric circuit theory and electromagnetic theory are the tw o funda -

mental theories upon which all branches of electrical engineering are

built. Many branches of electrical engineering, such as po wer, electric

machines, control, electronics, communications, and instrumentation,

are based on electric circuit theory . Therefore, the basic electric circuit

theory course is the most important course for an electrical engineering

student, and al ways an e xcellent starting point for a be ginning student

in electrical engineering education. Circuit theory is also v aluable to

students specializing in other branches of the ph ysical sciences because

circuits are a good model for the study of energy systems in general, and

because of the applied mathematics, physics, and topology involved.

In electrical engineering, we are often interested in communicating

or transferring energy from one point to another . To do this requires an

interconnection of electrical devices. Such interconnection is referred to

as an electric circuit, and each component of the circuit is kno wn as an

element.

An electric circuit is an interconnection of electrical elements.

A simple electric circuit is sho wn in Fig. 1.1. It consists of three

basic elements: a battery, a lamp, and connecting wires. Such a simple

circuit can e xist by itself; it has se veral applications, such as a flash

Current

+ –

light, a search light, and so forth.

A complicated real circuit is displayed in Fig. 1.2, representing the schematic diagram for a radio receiver. Although it seems complicated,  this circuit can be analyzed using the techniques we co ver in this book.

Battery Lamp

Figure 1.1

A simple electric circuit.

Our goal in this text is to learn various analytical techniques and computer  software applications for describing the behavior of a circuit like this. Electric circuits are used in numerous electrical systems to accomplish different tasks. Our objecti ve in this book is not the study of various uses  and applications of circuits. Rather, our major concern is the analysis of the  circuits. By the analysis of a circuit, we mean a study of the behavior of the

1.2 Systems of Units 5

Electret

microphone Figure 1.2

R1

+ –

C1

R2 R4 R6 C2

Q1

R3 R5

+ 9 V (DC) C3

L1 C4 Q C5 2

R7

Antenna

Electric circuit of a radio transmitter.

circuit: How does it respond to a gi ven input? How do the interconnected  elements and devices in the circuit interact?

We commence our study by defining some basic concepts. These  concepts include char ge, current, v oltage, circuit elements, po wer, and  energy. Before defining these concepts, we must first establish a system  of units that we will use throughout the text.

1.2 Systems of Units

As electrical engineers, we must deal with measurable quantities. Our mea surements, ho wever, must be communicated in a standard language that  virtually all professionals can understand, irrespecti ve of the country in  which the measurement is conducted. Such an international measurement language is the International System of Units (SI), adopted by the General  Conference on Weights and Measures in 1960. In this system, there are seven  base units from which the units of all other ph ysical quantities can be de - rived. Table 1.1 shows six base units and one derived unit (the coulomb) that  are related to this text. SI units are commonly used in electrical engineering.

One great advantage of the SI unit is that it uses prefixes based on the  power of 10 to relate larger and smaller units to the basic unit. Table 1.2  shows the SI prefixes and their symbols. For example, the following are  expressions of the same distance in meters (m):

600,000,000 mm 600,000 m 600 km

TABLE 1.1

Six basic SI units and one derived unit relevant to this text. Quantity Basic unit Symbol

Length meter m Mass kilogram kg Time second s Electric current ampere A Thermodynamic temperature kelvin K Luminous intensity candela cd Charge coulomb C

TABLE 1.2

The SI prefixes.

Multiplier Prefix Symbol 1018 exa E

1015 peta P

1012 tera T

109 giga G

106 mega M

103 kilo k

102 hecto h

10 deka da

10−1 deci d

10−2 centi c

10−3 milli m

10−6 micro μ

10−9 nano n

10−12 pico p

10−15 femto f

10−18 atto a

6 Chapter 1 Basic Concepts

1.3 Charge and Current

The concept of electric charge is the underlying principle for explaining

all electrical phenomena. Also, the most basic quantity in an electric cir

cuit is the electric charge. We all experience the effect of electric charge

when we try to remove our wool sweater and have it stick to our body or

walk across a carpet and receive a shock.

Charge is an electrical property of the atomic particles of which matter

consists, measured in coulombs (C).

We know from elementary physics that all matter is made of funda

mental building blocks known as atoms and that each atom consists of

electrons, protons, and neutrons. We also know that the char ge e on an

electron is negative and equal in magnitude to 1.602 × 10−19 C, while a

proton carries a positi ve charge of the same magnitude as the electron.

The presence of equal numbers of protons and electrons leaves an atom

neutrally charged.

The following points should be noted about electric charge:

1. The coulomb is a large unit for charges. In 1 C of charge, there are

1∕(1.602 × 10−19) = 6.24 × 1018 electrons. Thus realistic or labora

tory values of charges are on the order of pC, nC, or μC.1

2. According to e xperimental observ ations, the only char ges that

occur in nature are inte gral multiples of the electronic char ge

e = −1.602 × 10−19 C.

3. The law of conservation of charge states that charge can neither be

created nor destroyed, only transferred. Thus, the algebraic sum of

the electric charges in a system does not change.

We now consider the flow of electric char ges. A unique feature of

electric charge or electricity is the f act that it is mobile; that is, it can

be transferred from one place to another , where it can be con verted to

another form of energy.

When a conducting wire (consisting of se veral atoms) is connected

I

––

–

–

+ –

to a battery (a source of electromotive force), the charges are compelled  to move; positive charges move in one direction while ne gative charges  move in the opposite direction. This motion of char ges creates elec - tric current. It is conventional to take the current flow as the movement  of positive charges. That is, opposite to the flow of ne gative charges,

Battery

Figure 1.3

Electric current due to flow of electronic  charge in a conductor.

A convention is a standard way of   describing something so that others in  the profession can understand what  we mean. We will be using IEEE  conventions throughout this book.

as Fig.  1.3 illustrates. This con vention w as introduced by Benjamin  Franklin (1706–1790), the American scientist and in ventor. Although  we now know that current in metallic conductors is due to ne gatively  charged electrons, we will follo w the uni versally accepted con vention  that current is the net flow of positive charges. Thus,

Electric current is the time rate of change of charge, measured in  amperes (A).

Mathematically, the relationship between current i, charge q, and time t is i = Δ dq___

dt (1.1)

1 However, a large power supply capacitor can store up to 0.5 C of charge.

1.3 Charge and Current 7

Historical

Andre-Marie Ampere (1775–1836), a French mathematician and

physicist, laid the foundation of electrodynamics. He defined the electric

current and developed a way to measure it in the 1820s.

Born in Lyons, France, Ampere at age 12 mastered Latin in a few

weeks, as he was intensely interested in mathematics and many of the

best mathematical works were in Latin. He was a brilliant scientist and a

prolific writer. He formulated the laws of electromagnetics. He in vented

the electromagnet and the ammeter. The unit of electric current, the

ampere, was named after him.

© Apic/Getty Images

where current is measured in amperes (A), and

1 ampere = 1 coulomb/second

The charge transferred between time t0 and t is obtained by inte grating

both sides of Eq. (1.1). We obtain

Q = Δ ∫t0ti dt (1.2)

The way we define current as i in Eq. (1.1) suggests that current need not

be a constant-valued function. As many of the examples and problems in

this chapter and subsequent chapters suggest, there can be se veral types

of current; that is, charge can vary with time in several ways. There are different ways of looking at direct current and alternating  current. The best definition is that there are two ways that current can  flow: It can always flow in the same direction, where it does not reverse  direction, in which case we have direct current (dc). These currents can  be constant or time varying. If the current flows in both directions, then  we have alternating current (ac).

A direct current (dc) flows only in one direction and can be constant  or time varying.

By convention, we will use the symbol I to represent a constant current.  If the current v aries with respect to time (either dc or ac) we will use  the symbol i. A common use of this w ould be the output of a rectifier  (dc) such as i(t) = ∣5 sin(377t)∣ amps or a sinusoidal current (ac) such as  i(t) = 160 sin(377t) amps.

An alternating current (ac) is a current that changes direction with  respect to time.

An example of alternating current (ac) is the current you use in your  house to run the air conditioner , refrigerator , w ashing machine, and  other electric appliances. Figure 1.4 depicts tw o common e xamples of

I

0 t

(a)

i

0 t

(b)

Figure 1.4

Two common types of current: (a) direct  current (dc), (b) alternating current (ac).

8 Chapter 1 Basic Concepts

dc (coming from a battery) and ac (coming from your home outlets). We

5 A

(a)

Figure 1.5

–5 A

(b)

will consider other types later in the book.

Once we define current as the movement of charge, we expect cur rent to have an associated direction of flow. As mentioned earlier, the di rection of current flow is conventionally taken as the direction of positive  charge movement. Based on this convention, a current of 5 A may be rep resented positively or negatively as shown in Fig. 1.5. In other w ords, a

Conventional current flow: (a) positive  current flow, (b) negative current flow.

negative current of −5 A flowing in one direction as shown in Fig. 1.5(b)  is the same as a current of +5 A flowing in the opposite direction.

Example 1.1 How much charge is represented by 4,600 electrons?

Solution:

Each electron has −1.602 × 10 −19 C. Hence 4,600 electrons will

have −1.602 × 10−19 C/electron × 4,600 electrons = −7.369 × 10−16 C

Practice Problem 1.1

Calculate the amount of charge represented by 6.667 billion protons. Answer: 1.0681 × 10−9 C.

Example 1.2 The total char ge entering a terminal is gi ven by q = 5 t sin 4 πt mC.  Calculate the current at t = 0.5 s.

Solution:

i = dq___

dt = __ddt (5t sin 4πt) mC/s = (5 sin 4πt + 20πt cos 4πt) mA

At t = 0.5,

i = 5 sin 2π + 10π cos 2π = 0 + 10π = 31.42 mA

Practice Problem 1.2

If in Example 1.2, q = (10 − 10e−2t) mC, find the current at t = 1.0 s. Answer: 2.707 mA.

Example 1.3 Determine the total charge entering a terminal between t = 1 s and  t = 2 s if the current passing the terminal is i = (3t2 − t) A.

Solution:

2

Q = ∫t = 1

i dt = ∫12 (3t2 − t) dt

= (t3 − t2__2 )∣12= (8 − 2) − ( 1 − __12 ) = 5.5 C

The current flowing through an element is  i = {4 A, 0 < t < 1 4t2 A, t > 1

1.4 Voltage 9 Practice Problem 1.3

Calculate the charge entering the element from t = 0 to t = 2 s. Answer: 13.333 C.

1.4 Voltage

As explained briefly in the previous section, to mo ve the electron in a  conductor in a particular direction requires some w ork or energy trans fer. This work is performed by an e xternal electromotive force (emf),  typically represented by the battery in Fig. 1.3. This emf is also kno wn

as voltage or potential difference. The voltage vab between two points a and b in an electric circuit is the energy (or work) needed to move a unit  charge from b to a; mathematically,

vab =Δ ___ dw

dq (1.3)

where w is energy in joules (J) and q is charge in coulombs (C). The volt age vab or simply v is measured in volts (V), named in honor of the Italian  physicist Alessandro Antonio Volta (1745–1827), who invented the first  voltaic battery. From Eq. (1.3), it is evident that

1 volt = 1 joule/coulomb = 1 newton-meter/coulomb

Thus,

Voltage (or potential difference) is the energy required to move a  unit charge from a reference point (−) to another point (+), measured in  volts (V).

Figure 1.6 sho ws the v oltage across an element (represented by  a rectangular block) connected to points a and b. The plus ( +) and  minus (−) signs are used to define reference direction or voltage polarity.  The vab can be interpreted in two ways: (1) Point a is at a potential of vab volts higher than point b, or (2) the potential at point a with respect to  point b is vab. It follows logically that in general

a

+

vab

–

b

Figure 1.6

Polarity of voltage vab.

a

vab = −vba (1.4)

+

For example, in Fig. 1.7, we ha ve two representations of the same v olt 9 V

age. In Fig. 1.7(a), point a is +9 V above point b; in Fig. 1.7(b), point  b is −9 V above point a. We may say that in Fig. 1.7(a), there is a 9-V  –

voltage drop from a to b or equivalently a 9-V voltage rise from b to a.

b

In other words, a voltage drop from a to b is equivalent to a voltage rise  (a)

–

–9 V +

(b)

a b

from b to a.

Current and voltage are the tw o basic variables in electric circuits.  The common term signal is used for an electric quantity such as a current  or a voltage (or even electromagnetic wave) when it is used for conveying

Figure 1.7

Two equivalent representations of the

same voltage vab: (a) Point a is 9 V above  point b; (b) point b is −9 V above point a.

10 Chapter 1 Basic Concepts

Historical

Alessandro Antonio Volta (1745–1827), an Italian physicist, in -

vented the electric battery—which provided the first continuous flow of

electricity—and the capacitor.

Born into a noble family in Como, Italy, Volta was performing

electrical experiments at age 18. His invention of the battery in 1796

revolutionized the use of electricity. The publication of his work in 1800

marked the beginning of electric circuit theory. Volta received many

honors during his lifetime. The unit of voltage or potential difference,

the volt, was named in his honor.

© UniversalImagesGroup/

Getty Images

information. Engineers prefer to call such v ariables signals rather than

mathematical functions of time because of their importance in commu -

nications and other disciplines. Lik e electric current, a constant v oltage

Keep in mind that electric current is   always through an element and that  electric voltage is always across the   element or between two points.

is called a dc voltage and is represented by V, whereas a sinusoidally  time-varying voltage is called an ac voltage and is represented by v. A dc  voltage is commonly produced by a battery; ac v oltage is produced by  an electric generator.

1.5 Power and Energy

Although current and v oltage are the tw o basic v ariables in an electric  circuit, the y are not suf ficient by themselves. F or practical purposes,  we need to kno w how much power an electric de vice can handle. We  all know from e xperience that a 100-w att bulb gives more light than a  60-watt bulb. We also know that when we pay our bills to the electric  utility companies, we are paying for the electric energy consumed over  a certain period of time. Thus, power and energy calculations are impor tant in circuit analysis.

To relate power and ener gy to v oltage and current, we recall from  physics that:

Power is the time rate of expending or absorbing energy, measured in  watts (W).

We write this relationship as

p = Δ  ___ dwdt (1.5)

1.5 Power and Energy 11

where p is power in watts (W), w is energy in joules (J), and t is time in  seconds (s). From Eqs. (1.1), (1.3), and (1.5), it follows that

dq ⋅ dq__dt = vi (1.6)

p = ___ dwdt = ___ dw

or

i i +

v

–

+ v

–

p = vi (1.7)

The power p in Eq. (1.7) is a time-varying quantity and is called the instan

p = +vi

(a)

Figure 1.8

p = –vi (b)

taneous power. Thus, the power absorbed or supplied by an element is the  product of the voltage across the element and the current through it. If the  power has a + sign, power is being delivered to or absorbed by the element. If,  on the other hand, the power has a − sign, power is being supplied by the ele ment. But how do we know when the power has a negative or a positive sign?

Current direction and voltage polarity play a major role in determin ing the sign of po wer. It is therefore important that we pay attention to  the relationship between current i and voltage v in Fig. 1.8(a). The volt age polarity and current direction must conform with those sho wn in

Fig. 1.8(a) in order for the power to have a positive sign. This is known  as the passive sign convention. By the passive sign convention, current  enters through the positive polarity of the voltage. In this case, p = +vi or  vi > 0 implies that the element is absorbing po wer. However, if p = −vi

or vi < 0, as in Fig. 1.8(b), the element is releasing or supplying power. Passive sign convention is satisfied when the current enters through the

Reference polarities for power using the  passive sign convention: (a) absorbing  power, (b) supplying power.

When the voltage and current directions  conform to Fig. 1.8(b), we have the  active sign convention and p = +vi.

positive terminal of an element and p = +vi. If the current enters through  the negative terminal, p = −vi.

Unless otherwise stated, we will follow the passive sign convention

3 A

+

3 A

–

throughout this text. For example, the element in both circuits of Fig. 1.9  has an absorbing power of +12 W because a positive current enters the  positive terminal in both cases. In Fig. 1.10, ho wever, the element is  supplying power of +12 W because a positive current enters the nega tive terminal. Of course, an absorbing power of −12 W is equivalent to a  supplying power of +12 W. In general,

4 V

–

(a)

Figure 1.9

4 V

+

(b)

+Power absorbed = −Power supplied

In fact, the law of conservation of energy must be obeyed in any  electric circuit. For this reason, the algebraic sum of po wer in a circuit,

Two cases of an element with an absorb ing power of 12 W: (a) p = 4 × 3 = 12 W,  (b) p = 4 × 3 = 12 W.

3 A

3 A

at any instant of time, must be zero:

–

+

∑ p = 0 (1.8)

4 V

4 V

This again confirms the fact that the total po wer supplied to the circuit

+

–

must balance the total power absorbed.

From Eq. (1.6), the energy absorbed or supplied by an element from

(b)

(a)

time t0 to time t is

w = ∫t0tp dt = ∫t0tvi dt (1.9)

Figure 1.10

Two cases of an element with a supplying  power of 12 W: (a) p = −4 × 3 = −12 W,  (b) p = −4 × 3 = −12 W.

12 Chapter 1 Basic Concepts

Energy is the capacity to do work, measured in joules (J).

The electric po wer utility companies measure ener gy in w att-hours

(Wh), where

1 Wh = 3,600 J

Example 1.4 An energy source forces a constant current of 2 A for 10 s to flow through  a light bulb. If 2.3 kJ is gi ven off in the form of light and heat ener gy,

calculate the voltage drop across the bulb.

Solution:

The total charge is

∆q = i ∆t = 2 × 10 = 20 C

The voltage drop is

∆q = 2.3 × 103 _______

∆w

v = ___

20 = 115 V

Practice Problem 1.4

To move charge q from point b to point a requires 25 J. Find the volt - age drop vab (the voltage at a positive with respect to b) if: (a) q = 5 C,  (b) q = −10 C.

Answer: (a) 5 V, (b) −2.5 V.

Example 1.5 Find the power delivered to an element at t = 3 ms if the current entering  its positive terminal is

i = 5 cos 60π t A

and the voltage is: (a) v = 3i, (b) v = 3 di∕dt.

Solution:

(a) The voltage is v = 3i = 15 cos 60π t; hence, the power is

p = vi = 75 cos2 60π t W

At t = 3 ms,

p = 75 cos2 (60π × 3 × 10−3) = 75 cos2 0.18π = 53.48 W

(b) We find the voltage and the power as

v = 3 __didt = 3(−60π)5 sin 60π t = −900π sin 60π t V

p = vi = −4500π sin 60π t cos 60π t W

At t = 3 ms,

p = −4500π sin 0.18π cos 0.18π W

= −14137.167 sin 32.4° cos 32.4° = −6.396 kW

1.5 Power and Energy 13

Historical

1884 Exhibition In the United States, nothing promoted the future of

electricity like the 1884 International Electrical Exhibition. Just imagine

a world without electricity, a world illuminated by candles and gaslights,

a world where the most common transportation was by walking and rid

ing on horseback or by horse-drawn carriage. Into this world an exhibi -

tion was created that highlighted Thomas Edison and reflected his highly

developed ability to promote his inventions and products. His exhibit

featured spectacular lighting displays powered by an impressive 100-kW

“Jumbo” generator.

Edward Weston’s dynamos and lamps were featured in the United

States Electric Lighting Company’s display. Weston’s well known col -

lection of scientific instruments was also shown.

Other prominent exhibitors included Frank Sprague, Elihu Thompson,

and the Brush Electric Company of Cleveland. The American Institute of

Electrical Engineers (AIEE) held its first technical meeting on October 7–8

at the Franklin Institute during the exhibit. AIEE merged with the Institute

of Radio Engineers (IRE) in 1964 to form the Institute of Electrical and

Electronics Engineers (IEEE).

Source: IEEE History Center

Find the power delivered to the element in Example 1.5 at t = 5 ms if the  current remains the same but the voltage is: (a) v = 2i V, (b) v = ( 10 + 5 ∫0t i dt ) V.

Answer: (a) 17.27 W, (b) 29.7 W.

Practice Problem 1.5

14 Chapter 1 Basic Concepts

Example 1.6 How much energy does a 100-W electric bulb consume in two hours?

Solution:

w = pt = 100 (W) × 2 (h) × 60 (min/h) × 60 (s/min)

= 720,000 J = 720 kJ

This is the same as

w = pt = 100 W × 2 h = 200 Wh

Practice Problem 1.6+

A home electric heater dra ws 10 A when connected to a 115 V outlet.  How much energy is consumed by the heater over a period of 6 hours?

Answer: 6.9 k watt-hours

1.6 Circuit Elements

As we discussed in Section 1.1, an element is the basic building block of  a circuit. An electric circuit is simply an interconnection of the elements.  Circuit analysis is the process of determining voltages across (or the cur rents through) the elements of the circuit.

There are tw o types of elements found in electric circuits: pas sive elements and active elements. An active element is capable of  generating energy while a passive element is not. Examples of passive  elements are resistors, capacitors, and inductors. Typical active ele ments include generators, batteries, and operational amplifiers. Our aim in this section is to g ain familiarity with some important acti ve  elements.

The most important acti ve elements are v oltage or current sources  that generally deliver power to the circuit connected to them. There are  two kinds of sources: independent and dependent sources.

An ideal independent source is an active element that provides a  speci fied voltage or current that is completely independent of other  circuit elements.

In other words, an ideal independent voltage source delivers to the circuit

v

(a)

+–

V

–

(b)

whatever current is necessary to maintain its terminal v oltage. Physical  sources such as batteries and generators may be regarded as approxima tions to ideal v oltage sources. Figure 1.11 sho ws the symbols for inde -

pendent voltage sources. Notice that both symbols in Fig. 1.11(a) and  (b) can be used to represent a dc v oltage source, b ut only the symbol

Figure 1.11

Symbols for independent voltage sources:  (a) used for constant or time-varying volt age, (b) used for constant voltage (dc).

in Fig. 1.11(a) can be used for a time-v arying voltage source. Similar ly, an ideal independent current source is an active element that pro vides a specified current completely independent of the voltage across  the source. That is, the current source deli vers to the circuit whate ver

1.6 Circuit Elements 15

voltage is necessary to maintain the designated current. The symbol for

an independent current source is displayed in Fig. 1.12, where the arrow

indicates the direction of current i.

An ideal dependent (or controlled) source is an active element in  which the source quantity is controlled by another voltage or current.

Dependent sources are usually designated by diamond-shaped sym - bols, as shown in Fig. 1.13. Since the control of the dependent source is  achieved by a voltage or current of some other element in the circuit, and  the source can be voltage or current, it follows that there are four possible  types of dependent sources, namely:

1. A voltage-controlled voltage source (VCVS).

2. A current-controlled voltage source (CCVS).

3. A voltage-controlled current source (VCCS).

4. A current-controlled current source (CCCS).

Dependent sources are useful in modeling elements such as transistors,  operational amplifiers, and integrated circuits. An example of a current controlled voltage source is sho wn on the right-hand side of Fig. 1.14,  where the v oltage 10i of the v oltage source depends on the current i through element C. Students might be surprised that the value of the de pendent voltage source is 10 i V (and not 10 i A) because it is a v oltage

source. The key idea to keep in mind is that a voltage source comes with  polarities (+ −) in its symbol, while a current source comes with an ar row, irrespective of what it depends on.

It should be noted that an ideal v oltage source (dependent or in - dependent) will produce any current required to ensure that the termi - nal voltage is as stated, whereas an ideal current source will produce  the necessary voltage to ensure the stated current flow. Thus, an ideal  source could in theory supply an infinite amount of energy. It should  also be noted that not only do sources supply po wer to a circuit, the y

can absorb power from a circuit too. For a voltage source, we know the  voltage but not the current supplied or drawn by it. By the same token,  we know the current supplied by a current source b ut not the v oltage  across it.

i

Figure 1.12

Symbol for independent current source.

v +– i

(a) (b)

Figure 1.13

Symbols for: (a) dependent voltage  source, (b) dependent current source.

A B

i

+

5 V C 10i +– −

Figure 1.14

The source on the right-hand side is a   current-controlled voltage source.

Calculate the power supplied or absorbed by each element in Fig. 1.15. Example 1.7

Solution:

We apply the sign convention for power shown in Figs. 1.8 and 1.9. For

p1, the 5-A current is out of the positive terminal (or into the negative

I = 5 A

p2

terminal); hence,

p1 = 20(−5) = −100 W Supplied power

+–

20 V

+ – 12 V

p3

6 A +

For p2 and p3, the current flows into the positive terminal of the element  in each case.

p2 = 12(5) = 60 W Absorbed power

p3 = 8(6) = 48 W Absorbed power

p1 p4

8 V 0.2I

–

Figure 1.15

For Example 1.7.

16 Chapter 1 Basic Concepts

For p4, we should note that the voltage is 8 V (positive at the top), the

same as the voltage for p3 since both the passive element and the depen

dent source are connected to the same terminals. (Remember that v olt

age is always measured across an element in a circuit.) Since the current

flows out of the positive terminal,

p4 = 8(−0.2I ) = 8(−0.2 × 5) = −8 W Supplied power

We should observ e that the 20-V independent v oltage source and

0.2I dependent current source are supplying power to the rest of the net

work, while the two passive elements are absorbing power. Also,

p1 + p2 + p3 + p4 = −100 + 60 + 48 − 8 = 0

In agreement with Eq. (1.8), the total po wer supplied equals the total

power absorbed.

Practice Problem 1.7

Compute the power absorbed or supplied by each component of the cir cuit in Fig. 1.16.

9 A

+

2 V + – p2

I = 5 A

4 A

-

+–

Answer: p1 = −45 W, p2 = 18 W, p3 = 12 W, p4 = 15 W. +

p1 p3 p4

+ 0.6I

5 V 3 V

–

Figure 1.16

For Practice Prob. 1.7.

–

1.7 Applications2

In this section, we will consider tw o practical applications of the con - cepts developed in this chapter. The first one deals with the TV picture  tube and the other with how electric utilities determine your electric bill.

1.7.1 TV Picture Tube

One important application of the motion of electrons is found in both

the transmission and reception of TV signals. At the transmission end, a  TV camera reduces a scene from an optical image to an electrical signal.  Scanning is accomplished with a thin beam of electrons in an iconoscope  camera tube.

At the receiving end, the image is reconstructed by using a cathode

ray tube (CR T) located in the TV recei ver.3 The CRT is depicted in

Fig. 1.17. Unlike the iconoscope tube, which produces an electron beam  of constant intensity, the CRT beam varies in intensity according to the  incoming signal. The electron gun, maintained at a high potential, fires  the electron beam. The beam passes through two sets of plates for verti cal and horizontal deflections so that the spot on the screen where the  beam strikes can move right and left and up and down. When the electron  beam strikes the fluorescent screen, it gives off light at that spot. Thus,  the beam can be made to “paint” a picture on the TV screen.

2 The dagger sign preceding a section heading indicates the section that may be skipped,

explained briefly, or assigned as homework.

3 Modern TV tubes use a different technology.

Electron gun

(A)

Plates for

1.7 Applications 17

(B)

Plates for

horizontal deflection horizontal deflection

Plates for

Heated filament

(source of electrons)

– – – –

vertical deflection vertical deflection

s)

Cathode de

Anode

Anode

(–)

Figure 1.17

Cathode-ray tube.

(+)++

(+)++ Electron

Electron

beam

beam

Conductive coating

Conductive coating

Fluorescent screen

Historical Karl Ferdinand Braun and Vladimir K. Zworykin

Karl Ferdinand Braun (1850–1918), of the University of Strasbourg,

invented the Braun cathode-ray tube in 1879. This then became the ba

sis for the picture tube used for so many years for televisions. It is still

the most economical device today, although the price of flat-screen sys

tems is rapidly becoming competitive. Before the Braun tube could be

used in television, it took the inventiveness of Vladimir K. Zworykin

(1889–1982) to develop the iconoscope so that the modern television

would become a reality. The iconoscope developed into the orthicon and

the image orthicon, which allowed images to be captured and converted

into signals that could be sent to the television receiver. Thus, the televi

sion camera was born.

The electron beam in a TV picture tube carries 10 Example 1.8 15 electrons per second.  As a design engineer, determine the voltage Vo needed to accelerate the

electron beam to achieve 4 W.

Solution:

The charge on an electron is

e = −1.6 × 10−19 C

18 Chapter 1 Basic Concepts

If the number of electrons is n, then q = ne and

i

q

Vo

Figure 1.18

A simplified diagram of the cathode-ray  tube; for Example 1.8.

Practice Problem 1.8

i = dq__dt = e __dndt = (−1.6 × 10 −19)( 10 15) = −1.6 × 10 −4 A

The negative sign indicates that the current flows in a direction oppo site to electron flow as shown in Fig. 1.18, which is a simplified dia gram of the CRT for the case when the vertical deflection plates carry no  charge. The beam power is

p = Voi or Vo = p__i = _________ 4

1.6 × 10−4 = 25,000 V

Thus, the required voltage is 25 kV.

If an electron beam in a TV picture tube carries 10 13 electrons/second  and is passing through plates maintained at a potential difference of  30 kV, calculate the power in the beam.

Answer: 48 mW.

1.7.2 Electricity Bills

The second application deals with ho w an electric utility compan y  charges their customers. The cost of electricity depends upon the  amount of ener gy consumed in kilo watt-hours (kWh). (Other f actors  that affect the cost include demand and po wer factors; we will ignore  these for now.) However, even if a consumer uses no energy at all, there  is a minimum service char ge the customer must pay because it costs  money to stay connected to the po wer line. As ener gy consumption  increases, the cost per kWh drops. It is interesting to note the a ver age monthly consumption of household appliances for a family of five,  shown in Table 1.3.

TABLE 1.3

Typical average monthly consumption of household

appliances.

Appliance kWh consumed Appliance kWh consumed

Water heater 500 Washing machine 120

Freezer 100 Stove 100

Lighting 100 Dryer 80

Dishwasher 35 Microwave oven 25

Electric iron 15 Personal computer 12

TV 10 Radio 8

Toaster 4 Clock 2

1.8 Problem Solving 19

A homeowner consumes 700 kWh in January. Determine the electricity Example 1.9 bill for the month using the following residential rate schedule:

Base monthly charge of $12.00.

First 100 kWh per month at 16 cents/kWh.

Next 200 kWh per month at 10 cents/kWh.

Over 300 kWh per month at 6 cents/kWh.

Solution:

We calculate the electricity bill as follows.

Base monthly charge = $12.00

First 100 kWh @ $0.16/k Wh = $16.00

Next 200 kWh @ $0.10/k Wh = $20.00

Remaining 400 kWh @ $0.06/k Wh = $24.00

Total charge = $72.00

Average cost = ______________ $72 100 + 200 + 400 = 10.2 cents/kWh

Referring to the residential rate schedule in Example 1.9, calculate the  average cost per kWh if only 350 kWh are consumed in July when the  family is on vacation most of the time.

Answer: 14.571 cents/kWh.

1.8 Problem Solving

Although the problems to be solved during one’s career will vary in com plexity and magnitude, the basic principles to be follo wed remain the  same. The process outlined here is the one developed by the authors over  many years of problem solving with students, for the solution of engi -

neering problems in industry, and for problem solving in research. We will list the steps simply and then elaborate on them.

1. Carefully define the problem.

2. Present everything you know about the problem.

3. Establish a set of alternative solutions and determine the one that  promises the greatest likelihood of success.

4. Attempt a problem solution.

5. Evaluate the solution and check for accuracy.

6. Has the problem been solved satisfactorily? If so, present the solu tion; if not, then return to step 3 and continue through the process  again.

1. Carefully define the problem . This may be the most important  part of the process, because it becomes the foundation for all the rest  of the steps. In general, the presentation of engineering problems is

Practice Problem 1.9

20 Chapter 1 Basic Concepts

somewhat  incomplete. You must do all you can to make sure you

understand the problem as thoroughly as the presenter of the problem

understands it. Time spent at this point clearly identifying the problem

will save you considerable time and frustration later. As a student, you

can clarify a problem statement in a textbook by asking your professor.

A problem presented to you in industry may require that you consult

several individuals. At this step, it is important to develop questions that

need to be addressed before continuing the solution process. If you have

such questions, you need to consult with the appropriate individuals or

resources to obtain the answers to those questions. With those answers,

you can now refine the problem, and use that refinement as the problem

statement for the rest of the solution process.

2. Present everything you know about the problem. You are now ready

to write down everything you know about the problem and its possible

solutions. This important step will save you time and frustration later.

3. Establish a set of alternative solutions and determine the one that

promises the greatest likelihood of success . Almost every problem

will have a number of possible paths that can lead to a solution. It is

highly desirable to identify as many of those paths as possible. At this

point, you also need to determine what tools are available to you, such

as PSpice and MATLAB and other software packages that can greatly

reduce effort and increase accuracy. Again, we want to stress that time

spent carefully defining the problem and investigating alternative ap-

proaches to its solution will pay big dividends later. Evaluating the al-

ternatives and determining which promises the greatest likelihood of

success may be difficult but will be well worth the effort. Document this

process well since you will want to come back to it if the first approach

does not work.

4. Attempt a problem solution. Now is the time to actually begin solv

ing the problem. The process you follow must be well documented in

order to present a detailed solution if successful, and to evaluate the

process if you are not successful. This detailed evaluation may lead to

corrections that can then lead to a successful solution. It can also lead to

new alternatives to try. Many times, it is wise to fully set up a solution

before putting numbers into equations. This will help in checking your

results.

5. Evaluate the solution and check for accuracy. You now thoroughly

evaluate what you have accomplished. Decide if you have an acceptable

solution, one that you want to present to your team, boss, or professor.

6. Has the problem been solved satisfactorily? If so, present the solu-

tion; if not, then return to step 3 and continue through the process again.

Now you need to present your solution or try another alternative. At this

point, presenting your solution may bring closure to the process. Often,

however, presentation of a solution leads to further refinement of the

problem definition, and the process continues. Following this process

will eventually lead to a satisfactory conclusion.

Now let us look at this process for a student taking an electrical and

computer engineering foundations course. (The basic process also ap-

plies to almost e very engineering course.) K eep in mind that although

the steps have been simplified to apply to academic types of problems,

the process as stated always needs to be followed. We consider a simple

example.

1.8 Problem Solving 21

Solve for the current flowing through the 8-Ω resistor in Fig. 1.19. Example 1.10 Solution:

1. Carefully define the problem. This is only a simple example, but we  can already see that we do not know the polarity on the 3-V source. We have the following options. We can ask the professor what the polarity  should be. If we cannot ask, then we need to make a decision on what  to do next. If we have time to work the problem both ways, we can  solve for the current when the 3-V source is plus on top and then plus on the bottom. If we do not have the time to work it both ways, assume a polarity and then carefully document your decision. Let us assume  that the professor tells us that the source is plus on the bottom as shown  in Fig. 1.20.

2. Present everything you know about the problem. Presenting all that  we know about the problem involves labeling the circuit clearly so that  we define what we seek.

Given the circuit shown in Fig. 1.20, solve for i8Ω.

We now check with the professor , if reasonable, to see if the prob - lem is properly defined.

3. Establish a set of alternative solutions and determine the one that  promises the greatest likelihood of success . There are essentially three  techniques that can be used to solve this problem. Later in the text you  will see that you can use circuit analysis (using Kirchhoff’s laws and  Ohm’s law), nodal analysis, and mesh analysis.

To solve for i8Ω using circuit analysis will eventually lead to a solu tion, but it will likely take more work than either nodal or mesh analysis.  To solve for i8Ω using mesh analysis will require writing two simultane ous equations to find the two loop currents indicated in Fig. 1.21. Using  nodal analysis requires solving for only one unknown. This is the easiest  approach.

i1 i3

2 Ω 4 Ω

v1

+ – + –

v v2Ω 4Ω

i2

5 V 3 V +– +–

+

Loop 1 Loop 2

2 Ω 4 Ω

5 V 3 8 Ω V +–

Figure 1.19

Illustrative example.

2 Ω 4 Ω

i8 Ω

5 V 3 8 Ω V +– +–

Figure 1.20

Problem definition.

v8Ω

–

Figure 1.21

Using nodal analysis.

8 Ω

Therefore, we will solve for i8Ω using nodal analysis.

4. Attempt a problem solution. We first write down all of the equations

we will need in order to find i8Ω.

i 8Ω = i 2 , i 2 = v __18 , i 8Ω = v __18

v ____ 1 − 5

2 + v ____ 1 − 0

8 + v ____ 1 + 3

4 = 0

22 Chapter 1 Basic Concepts

Now we can solve for v1.

8 [ v1 − 5 ______

2 + v1 − 0 ______

8 + v1 + 3 ______

4 ] = 0

leads to (4 v 1 − 20) + (v 1) + (2 v 1 + 6) = 0

7v1 = +14, v 1 = +2 V, i 8Ω = v __18 = __28 = 0.25 A

5. Evaluate the solution and check for accuracy . We can now use

Kirchhoff’s voltage law (KVL) to check the results.

i 1 = v 1 − 5 ____

2 = 2 ___− 5

2 = − __32 = −1.5 A

i2 = i8Ω = 0.25 A

i 3 = v ____ 1 + 3

4 = ___ 2 + 3

4 = __54 = 1.25 A

i1 + i2 + i3 = −1.5 + 0.25 + 1.25 = 0 (Checks.)

Applying KVL to loop 1,

−5 + v 2Ω + v 8Ω = −5 + (−i 1 × 2) + (i 2 × 8)

= −5 + [−(−1.5) 2] + (0.25 × 8)

= −5 + 3 + 2 = 0 (Checks.)

Applying KVL to loop 2,

−v 8Ω + v 4Ω − 3 = −(i 2 × 8) + (i 3 × 4) − 3

= −(0.25 × 8) + (1.25 × 4) − 3

= −2 + 5 − 3 = 0 (Checks.)

So we now have a v ery high de gree of confidence in the accuracy

of our answer.

6. Has the problem been solved satisfactorily? If so, present the solu

tion; if not, then return to step 3 and continue through the process again.

This problem has been solved satisfactorily.

The current through the 8-Ω resistor is 0.25 A flowing down through the

8-Ω resistor.

Practice Problem 1.10

Try applying this process to some of the more difficult problems at the  end of the chapter.

1.9 Summary

1. An electric circuit consists of electrical elements connected together. 2. The International System of Units (SI) is the international mea - surement language, which enables engineers to communicate their

results. From the se ven principal units, the units of other ph ysical  quantities can be derived.

Review Questions 23

3. Current is the rate of char ge flow past a gi ven point in a gi ven

direction.

i = dq___

dt

4. Voltage is the energy required to move 1 C of char ge from a refer

ence point (−) to another point (+).

vab = ___ dwdq

5. Power is the energy supplied or absorbed per unit time. It is also the

product of voltage and current.

p = ___ dwdt = vi

6. According to the passi ve sign con vention, power assumes a posi -

tive sign when the current enters the positive polarity of the voltage

across an element.

7. An ideal v oltage source produces a specific potential difference

across its terminals re gardless of what is connected to it. An ideal

current source produces a specific current through its terminals

regardless of what is connected to it.

8. Voltage and current sources can be dependent or independent. A

dependent source is one whose value depends on some other circuit

variable.

9. Two areas of application of the concepts covered in this chapter are

the TV picture tube and electricity billing procedure.

Review Questions

1.1 One millivolt is one millionth of a volt. (a) True (b) False

1.2 The prefix micro stands for:

(a) 106 (b) 103 (c) 10−3 (d) 10−6

1.3 The voltage 2,000,000 V can be expressed in powers  of 10 as:

(a) 2 mV (b) 2 kV (c) 2 MV (d) 2 GV

1.4 A charge of 2 C flowing past a given point each  second is a current of 2 A.

(a) True (b) False

1.5 The unit of current is:

(a) coulomb (b) ampere

1.8 The voltage across a 1.1-kW toaster that produces a  current of 10 A is:

(a) 11 kV (b) 1100 V (c) 110 V (d) 11 V 1.9 Which of these is not an electrical quantity?

(a) charge (b) time (c) voltage

(d) current (e) power

1.10 The dependent source in Fig. 1.22 is:

(a) voltage-controlled current source

(b) voltage-controlled voltage source

(c) current-controlled voltage source

(d) current-controlled current source

io

(c) volt (d) joule

vs

1.6 Voltage is measured in:

6io +–

(a) watts (b) amperes

(c) volts (d) joules per second

1.7 A 4-A current charging a dielectric material will  accumulate a charge of 24 C after 6 s.

(a) True (b) False

Figure 1.22

For Review Question 1.10.

Answers: 1.1b, 1.2d, 1.3c, 1.4a, 1.5b, 1.6c, 1.7a, 1.8c,  1.9b, 1.10d.

24 Chapter 1 Basic Concepts

Problems

Section 1.3 Charge and Current

1.1 How much charge is represented by these number of  electrons?

(a) 6.482 × 1017

(b) 1.24 × 1018

(c) 2.46 × 1019

q (C)

10

01 2 3 4 –10

t (s)

(d) 1.628 × 1020

1.2 Determine the current flowing through an element if  the charge flow is given by

(a) q(t) = (3) mC

(b) q(t) = (4t2 + 20t − 4) C

(c) q(t) = (15e−3t − 2e−18t) nC

(d) q(t) = 5t2(3t3 + 4) pC

(e) q(t) = 2e−3t sin (20π t) µC

1.3 Find the charge q(t) flowing through a device if the  current is:

(a) i(t) = 3 A, q(0) = 1 C

(b) i (t) = (2t + 5) mA, q (0) = 0

(c) i (t) = 20 cos (10t + π∕6) μA, q (0) = 2 μC

(d) i (t) = 10 e −30t sin 40t A, q (0) = 0

1.4 A total charge of 300 C flows past a given cross  section of a conductor in 30 seconds. What is the  value of the current?

1.5 Determine the total charge transferred over the time  interval of 0 ≤ t ≤ 10 s when i(t) = __12 t A.

1.6 The charge entering a certain element is shown in

Figure 1.24

For Prob. 1.7.

1.8 The current flowing past a point in a device is shown  in Fig. 1.25. Calculate the total charge through the  point.

i (mA)

10

0 1 2 t (ms)

Figure 1.25

For Prob. 1.8.

1.9 The current through an element is shown in  Fig. 1.26. Determine the total charge that passed  through the element at:

(a) t = 1 s (b) t = 3 s (c) t = 5 s

i (A)

10

5

Fig. 1.23. Find the current at:

(a) t = 1 ms (b) t = 6 ms (c) t = 10 ms

0 1 2 3 4 5

Figure 1.26

For Prob. 1.9.

t (s)

q(t) (mC)

30

Figure 1.23 For Prob. 1.6.

0 2 4 6 8 10 12 t (ms)

Sections 1.4 and 1.5 Voltage, Power, and Energy

1.10 A lightning bolt with 10 kA strikes an object for  15 μs. How much charge is deposited on the object?

1.11 A rechargeable flashlight battery is capable of deliv ering 90 mA for about 12 h. How much charge can  it release at that rate? If its terminal voltage is 1.5 V,  how much energy can the battery deliver?

1.12 If the current flowing through an element is given by i (t) ={3tA, 0 ≦ t < 6 s

18A, 6 ≦ t < 10 s

1.7 The charge flowing in a wire is plotted in Fig. 1.24.  Sketch the corresponding current.

−12A, 10 ≦ t < 15 s

0, t ≧ 15 s

Plot the charge stored in the element over 0 < t < 20 s.

1.13 The charge entering the positive terminal of an  element is

q = 5 sin 4 π t mC

while the voltage across the element (plus to  minus) is

Problems 25

1.17 Figure 1.28 shows a circuit with four elements,  p1 = 60 W absorbed, p3 = −145 W absorbed, and  p4 = 75 W absorbed. How many watts does element  2 absorb?

3

v = 3 cos 4 π t V

(a) Find the power delivered to the element at  t = 0.3 s.

(b) Calculate the energy delivered to the element  between 0 and 0.6 s.

4

1 2

Figure 1.28

For Prob. 1.17.

1.14 The voltage v(t) across a device and the current i(t)  through it are

v(t) = 20 sin (4t) V and i(t) = 10(1 + e−2t) mA Calculate:

(a) the total charge in the device at t = 1 s, q(0) = 0.

1.18 Find the power absorbed by each of the elements in  Fig. 1.29.

+ – 8 V

10 V

I = 10 A 4 A

+ –

p2 p4

14 A

0.4I 30 V + 20 V –+–12 V+–

(b) the power consumed by the device at t = 1 s.

1.15 The current entering the positive terminal of a  device is i(t) = 6e−2t mA and the voltage across the  device is v(t) = 10di∕dtV.

(a) Find the charge delivered to the device between  t = 0 and t = 2 s.

p1 p3

Figure 1.29

For Prob. 1.18.

p5

(b) Calculate the power absorbed.

(c) Determine the energy absorbed in 3 s.

Section 1.6 Circuit Elements

1.16 Figure 1.27 shows the current through and the

1.19 Find I and the power absorbed by each element in  the network of Fig. 1.30.

4 A I

−

−

9 V

voltage across an element.

+ − 10 A 15 V 15 V

(a) Sketch the power delivered to the element  for t > 0.

(b) Fnd the total energy absorbed by the element for  the period of 0 < t < 4s.

i (mA)

+

Figure 1.30 For Prob. 1.19.

+

6 V –+

60

0 2 4

t (s)

1.20 Find Vo and the power absorbed by each element in  the circuit of Fig. 1.31.

Io = 2 A

+ –

v (V) 5

6 A

12 V

+ –

1 A

3 A

+

28 V

+ – 28 V

30 V –+

00 4 2 –5

+–

t (s)

6 A

Vo 5Io –

3 A

Figure 1.27 For Prob. 1.16.

Figure 1.31 For Prob. 1.20.

26 Chapter 1 Basic Concepts

Section 1.7 Applications

1.21 A 60-W incandescent bulb operates at 120 V. How  many electrons and coulombs flow through the bulb

(b) how much energy is expended?

(c) how much does the charging cost? Assume  electricity costs 9 cents/kWh.

in one day?

1.22 A lightning bolt strikes an airplane with 40 kA for  1.7 ms. How many coulombs of charge are depos ited on the plane?

1.23 A 1.8-kW electric heater takes 15 min to boil a  quantity of water. If this is done once a day and  power costs 10 cents/kWh, what is the cost of its  operation for 30 days?

1.24 A utility company charges 8.2 cents/kWh. If a con sumer operates a 60-W light bulb continuously for  one day, how much is the consumer charged?

1.25 A 1.2-kW toaster takes roughly 4 minutes to heat  four slices of bread. Find the cost of operating the  toaster twice per day for 2 weeks (14 days). Assume  energy costs 9 cents/kWh.

1.26 A cell phone battery is rated at 3.85 V and can store  10.78 watt-hours of energy.

(a) How much average current can it deliver over a  period of 3 hours if it is fully discharged at the  end of that time?

(b) How much average power is delivered in  part (a)?

(c) What is the ampere-hour rating of the battery?

1.27 A constant current of 3 A for 4 hours is required  to charge an automotive battery. If the terminal volt age is 10 + t∕2V, where t is in hours,

(a) how much charge is transported as a result of the  charging?

Comprehensive Problems

1.32 A telephone wire has a current of 20μA flowing  through it. How long does it take for a charge of  15 C to pass through the wire?

1.33 A lightning bolt carried a current of 2 kA and lasted  for 3 ms. How many coulombs of charge were con tained in the lightning bolt?

1.34 Figure 1.32 shows the power consumption of a  certain household in 1 day. Calculate:

(a) the total energy consumed in kWh,

(b) the average power over the total 24 hour period.

1.28 A 150-W incandescent outdoor lamp is connected to  a 120-V source and is left burning continuously for

an average of 12 hours per day. Determine:

(a) the current through the lamp when it is lit.

(b) the cost of operating the light for one non-leap

year if electricity costs 9.5 cents per kWh.

1.29 An electric stove with four burners and an oven is

used in preparing a meal as follows.

Burner 1: 20 minutes Burner 2: 40 minutes

Burner 3: 15 minutes Burner 4: 45 minutes

Oven: 30 minutes

If each b urner is rated at 1.2 kW and the o ven at

1.8 kW, and electricity costs 12 cents per kWh, calcu

late the cost of electricity used in preparing the meal.

1.30 Reliant Energy (the electric company in Houston,

Texas) charges customers as follows:

Monthly charge $6

First 250 kWh @ $0.02/kWh

All additional kWh @ $0.07/kWh

If a customer uses 2,436 kWh in one month, ho w

much will Reliant Energy charge?

1.31 In a household, a business is run for an average of

6 h/day. The total power consumed by the computer

and its printer is 230 W. In addition, a 75-W light

runs during the same 6 h. If their utility charges

11.75 cents per kWh, how much do the owners pay

every 30 days?

1200 W

p

800 W

200 W

t (h)

12 2 4 6 8 10 12 2 4 6 10 128

noon

Figure 1.32

For Prob. 1.34.

Comprehensive Problems 27

1.35 The graph in Fig. 1.33 represents the power drawn  by an industrial plant between 8:00 and 8:30 a.m.  Cal culate the total energy in MWh consumed by the  plant.

p (MW)

8

543

voltage of 12 V. If an automobile battery is rated at  20 Ah:

(a) What is the maximum current that can be  supplied for 15 minutes?

(b) How many days will it last if it is discharged at a  rate of 2 mA?

1.37 A total of 2 MJ are delivered to an automobile  battery (assume 12 V) giving it an additional charge.  How much is that additional charge? Express your

8.00 8.05 8.10 8.15 8.20 8.25 8.30

t

Figure 1.33

For Prob. 1.35.

answer in ampere-hours.

1.38 How much energy does a 10-hp motor deliver in  30 minutes? Assume that 1 horsepower = 746 W.

1.36 A battery can be rated in ampere-hours (Ah) or watt  hours (Wh). The ampere hours can be obtained from  the watt hours by dividing watt hours by a nominal

1.39 A 600-W TV receiver is turned on for 4 h with no body watching it. If electricity costs 10 cents/kWh,  how much money is wasted?

This page intentionally left blank

chapter

Basic Laws 2

There are too many people praying for mountains of difficulty to be

removed, when what they really need is the courage to climb them!

—Unknown

Enhancing Your Skills and Your Career

ABET EC 2000 criteria (3.b), “an ability to design and con

duct experiments, as well as to analyze and interpret data.”

Engineers must be able to design and conduct e xperiments, as well as

analyze and interpret data. Most students ha ve spent man y hours per -

forming experiments in high school and in college. During this time, you

have been asked to analyze the data and to interpret the data. Therefore,

you should already be skilled in these tw o activities. My recommenda

tion is that, in the process of performing e xperiments in the future, you

spend more time in analyzing and interpreting the data in the conte xt of

the experiment. What does this mean?

If you are looking at a plot of voltage versus resistance or current

versus resistance or power versus resistance, what do you actually see?

Does the curve make sense? Does it agree with what the theory tells you?

Does it differ from e xpectation, and, if so, wh y? Clearly, practice with

analyzing and interpreting data will enhance this skill.

Since most, if not all, the e xperiments you are required to do as a

student involve little or no practice in designing the experiment, how can

you develop and enhance this skill?

Actually, developing this skill under this constraint is not as difficult

as it seems. What you need to do is to take the experiment and analyze it.

Just break it down into its simplest parts, reconstruct it trying to under -

stand why each element is there, and finally, determine what the author

of the experiment is trying to teach you. Even though it may not always

seem so, every experiment you do w as designed by someone who w as

sincerely motivated to teach you something.

29

30 Chapter 2 Basic Laws

Learning Objectives

By using the information and exercises in this chapter you will be

able to:

1. Know and understand the voltage current relationship of resis

tors (Ohm’s law).

2. Understand the basic structure of electrical circuits, essentially

nodes, loops, and branches.

3. Understand Kirchhoff’s voltage and current laws and their

importance in analyzing electrical circuits.

4. Understand series resistances and voltage division, and parallel

resistances and current division.

5. Know how to convert delta-connected circuits to wye-connected

circuits and how to convert wye-connected circuits to delta

connected circuits.

2.1 Introduction

Chapter 1 introduced basic concepts such as current, voltage, and power

in an electric circuit. To actually determine the v alues of these v ari

ables in a gi ven circuit requires that we understand some fundamen -

tal laws that govern electric circuits. These laws, known as Ohm’s law

and Kirchhoff’s laws, form the foundation upon which electric circuit

analysis is built.

In this chapter , in addition to these la ws, we shall discuss some

techniques commonly applied in circuit design and analysis. These tech

niques include combining resistors in series or parallel, voltage division,

current division, and delta-to-wye and wye-to-delta transformations. The

application of these laws and techniques will be restricted to resistive

circuits in this chapter. We will finally apply the laws and techniques to

real-life problems of electrical lighting and the design of dc meters.

2.2 Ohm’s Law

l

Cross-sectional area A

(a)

Figure 2.1

Material with resistivity ρ

i

+

v R –

(b)

Materials in general ha ve a characteristic beha vior of resisting the flow  of electric charge. This physical property, or ability to resist current, is  known as resistance and is represented by the symbol R. The resistance  of any material with a uniform cross-sectional area A depends on A and  its length ℓ, as sho wn in Fig. 2.1(a). We can represent resistance (as

measured in the laboratory), in mathematical form,

R = ρ __ℓA (2.1)

where ρ is known as the resistivity of the material in ohm-meters. Good  conductors, such as copper and aluminum, ha ve low resistivities, while

(a) Resistor, (b) Circuit symbol for  resistance.

insulators, such as mica and paper, have high resistivities. Table 2.1 pres ents the values of ρ for some common materials and shows which mate rials are used for conductors, insulators, and semiconductors.

The circuit element used to model the current-resisting beha vior of a  material is the resistor. For the purpose of constructing circuits, resistors are

TABLE 2.1

Resistivities of common materials.

2.2 Ohm’s Law 31

Material Resistivity (Ω∙m) Usage Silver 1.64 ×​10−8 Conductor Copper 1.72 ×​10−8 Conductor Aluminum 2.8 ×​10−8 Conductor Gold 2.45 ×​10−8 Conductor Carbon 4 ×​10−5 Semiconductor Germanium 47 ×​10−2 Semiconductor Silicon 6.4 ×​102 Semiconductor Paper 1010 Insulator Mica 5 ×​1011 Insulator Glass 1012 Insulator Teflon 3 ×​1012 Insulator

usually made from metallic alloys and carbon compounds. The circuit sym bol for the resistor is shown in Fig. 2.1(b), where R stands for the resistance  of the resistor. The resistor is the simplest passive element.

Georg Simon Ohm (1787–1854), a German ph ysicist, is credited  with finding the relationship between current and voltage for a resistor.  This relationship is known as Ohm’s law.

Ohm’s law states that the voltage v across a resistor is directly propor tional to the current i flowing through the resistor.

That is,

v ∝ i (2.2)

Ohm defined the constant of proportionality for a resistor to be the resis tance, R. (The resistance is a material property which can change if the  internal or external conditions of the element are altered, e.g., if there are  changes in the temperature.) Thus, Eq. (2.2) becomes

v = iR (2.3)

Historical

Georg Simon Ohm (1787–1854), a German physicist, in 1826  experimentally determined the most basic law relating voltage and cur - rent for a resistor. Ohm’s work was initially denied by critics.

Born of humble beginnings in Erlangen, Bavaria, Ohm threw himself  into electrical research. His efforts resulted in his famous law. He was  awarded the Copley Medal in 1841 by the Royal Society of London. In  1849, he was given the Professor of Physics chair by the University of  Munich. To honor him, the unit of resistance was named the ohm.

© SSPL via Getty Images

32 Chapter 2 Basic Laws

which is the mathematical form of Ohm’s law. R in Eq. (2.3) is mea sured

+

v = 0

–

+

v

–

Figure 2.2

(a) (b)

i

R = 0

i = 0

R = ∞

in the unit of ohms, designated Ω. Thus,

The resistance R of an element denotes its ability to resist the flow of  electric current; it is measured in ohms (Ω).

We may deduce from Eq. (2.3) that

R = __vi (2.4)

so that

1 Ω = 1 V/A

To apply Ohm’ s la w as stated in Eq. (2.3), we must pay careful  attention to the current direction and v oltage polarity. The direction of  current i and the polarity of voltage v must conform with the passive sign  convention, as shown in Fig. 2.1(b). This implies that current flows from  a higher potential to a lower potential in order for v = i R. If current flows  from a lower potential to a higher potential, v = −i R.

Since the value of R can range from zero to infinity, it is important

(a) Short circuit (R =​0), (b) Open circuit  (R =​∞).

(a)

(b)

Figure 2.3

Fixed resistors: (a) wirewound type,  (b) carbon film type.

© McGraw-Hill Education/Mark Dierker,  photographer

(a) (b)

Figure 2.4

Circuit symbol for: (a) a variable resistor  in general, (b) a potentiometer.

that we consider the tw o extreme possible values of R. An element with  R = 0 is called a short circuit, as shown in Fig. 2.2(a). For a short circuit,

v = i R = 0 (2.5)

showing that the v oltage is zero b ut the current could be an ything. In  practice, a short circuit is usually a connecting wire assumed to be a  perfect conductor. Thus,

A short circuit is a circuit element with resistance approaching zero.

Similarly, an element with R =​∞ is known as an open circuit, as shown  in Fig. 2.2(b). For an open circuit,

i =   lim

R→∞​​__vR =​0 (2.6)

indicating that the current is zero though the v oltage could be anything.  Thus,

An open circuit is a circuit element with resistance approaching infinity.

A resistor is either fixed or v ariable. Most resistors are of the fixed  type, meaning their resistance remains constant. The two common types  of fixed resistors (wirewound and composition) are shown in Fig. 2.3. The  composition resistors are used when large resistance is needed. The circuit  symbol in Fig. 2.1(b) is for a fixed resistor. Variable resistors have adjust

able resistance. The symbol for a variable resistor is shown in Fig. 2.4(a).  A common variable resistor is known as a potentiometer or pot for short,  with the symbol shown in Fig. 2.4(b). The pot is a three-terminal element  with a sliding contact or wiper . By sliding the wiper , the resistances be -

tween the wiper terminal and the fixed terminals v ary. Like fixed resis tors, variable resistors can be of either wire wound or composition type,  as shown in Fig. 2.5. Although resistors like those in Figs. 2.3 and 2.5 are  used in circuit designs, today most circuit components including resistors

are either surface mounted or integrated, as typically shown in Fig. 2.6.

2.2 Ohm’s Law 33

(a) (b)

Figure 2.5

Variable resistors: (a) composition type, (b) slider pot.

© McGraw-Hill Education/Mark Dierker, photographer

It should be pointed out that not all resistors obe y Ohm’s law. A

© Eric Tomey/Alamy RF

resistor that obeys Ohm’s law is known as a linear resistor. It has a con

stant resistance and thus its current-voltage characteristic is as illustrated

in Fig. 2.7(a): Its i-v graph is a straight line passing through the ori -

gin. A nonlinear resistor does not obe y Ohm’s law. Its resistance varies

with current and its i-v characteristic is typically sho wn in Fig. 2.7(b).

Examples of devices with nonlinear resistance are the light bulb and the  diode. Although all practical resistors may e xhibit nonlinear beha vior  under certain conditions, we will assume in this book that all elements  actually designated as resistors are linear.

A useful quantity in circuit analysis is the reciprocal of resistance R,  known as conductance and denoted by G:

G = __1R = __iv(2.7)

The conductance is a measure of how well an element will conduct  electric current. The unit of conductance is the mho (ohm spelled back ward) or reciprocal ohm, with symbol ℧, the inverted omega. Although  engineers often use the mho, in this book we prefer to use the siemens  (S), the SI unit of conductance:

1 S = 1 ℧ = 1 A / V (2.8)

Thus,

Conductance is the ability of an element to conduct electric current; it is  measured in mhos (℧) or siemens (S).

The same resistance can be e xpressed in ohms or siemens. F or  example, 10 Ω is the same as 0.1 S. From Eq. (2.7), we may write

i = Gv (2.9)

The power dissipated by a resistor can be e xpressed in terms of R.  Using Eqs. (1.7) and (2.3),

p = vi = i 2R = v 2 __R (2.10)

Figure 2.6

Resistors in an integrated circuit board.  v

Slope = R

i

(a)

v

Slope = R

i

(b)

Figure 2.7

The i-v characteristic of: (a) a linear

resistor, (b) a nonlinear resistor.

34 Chapter 2 Basic Laws

The power dissipated by a resistor may also be e xpressed in terms of

G as

p = vi = v2G = i2

__G (2.11)

We should note two things from Eqs. (2.10) and (2.11):

1. The power dissipated in a resistor is a nonlinear function of either

current or voltage.

2. Since R and G are positive quantities, the power dissipated in a resis

tor is al ways positive. Thus, a resistor al ways absorbs power from

the circuit. This confirms the idea that a resistor is a passive element,

incapable of generating energy.

Example 2.1

Practice Problem 2.1

Example 2.2

i

+

An electric iron draws 2 A at 120 V. Find its resistance.

Solution:

From Ohm’s law,

R = _vi = ___ 1202 = 60 Ω

The essential component of a toaster is an electrical element (a resis - tor) that converts electrical energy to heat energy. How much current is  drawn by a toaster with resistance 15 Ω at 110 V?

Answer: 7.333 A.

In the circuit shown in Fig. 2.8, calculate the current i, the conductance  G, and the power p.

Solution:

30 V

Figure 2.8

+– 5 kΩ v –

The voltage across the resistor is the same as the source voltage (30 V)  because the resistor and the voltage source are connected to the same  pair of terminals. Hence, the current is

i = __vR = _____ 30

For Example 2.2.

5 × 10 3 = 6 mA

The conductance is

G = __1R = _____ 1

5 × 10 3 = 0.2 mS

We can calculate the power in various ways using either Eqs. (1.7),  (2.10), or (2.11).

p = vi = 30 (6 × 10 −3) = 180 mW

or

p = i2R = (6 × 10 −3)2 5 × 10 3 = 180 mW

or

p = v 2G = (30)2 0.2 × 10 −3 = 180 mW

2.3 Nodes, Branches, and Loops 35

For the circuit shown in Fig. 2.9, calculate the voltage v, the conductance  G, and the power p.

Answer: 30 V, 100 µS, 90 mW.

Practice Problem 2.2

i

+

3 mA

Figure 2.9

For Practice Prob. 2.2

10 kΩ v –

A voltage source of 20 sin πt V is connected across a 5-kΩ resistor. Find Example 2.3 the current through the resistor and the power dissipated.

Solution:

i = __vR = _______ 20 sinπt

5 × 10 3 = 4 sinπt mA

Hence,

p = vi = 80 sin 2 π t mW

A resistor absorbs an instantaneous power of 30 cos Practice Problem 2.3 2 t mW when con - nected to a voltage source v = 15 cos t V. Find i and R.

Answer: 2 cos t mA, 7.5 kΩ.

2.3 Nodes, Branches, and Loops

Since the elements of an electric circuit can be interconnected in se veral

ways, we need to understand some basic concepts of network topology. To

differentiate between a circuit and a network, we may regard a network as

an interconnection of elements or devices, whereas a circuit is a network

providing one or more closed paths. The convention, when addressing

network topology, is to use the w ord network rather than circuit. We do

this even though the word network and circuit mean the same thing when

used in this context. In network topology, we study the properties relating

to the placement of elements in the network and the geometric configura

tion of the network. Such elements include branches, nodes, and loops.

A branch represents a single element such as a voltage source or a resistor.

In other words, a branch represents an y two-terminal element. The cir cuit in Fig. 2.10 has five branches, namely, the 10-V voltage source, the  2-A current source, and the three resistors.

A node is the point of connection between two or more branches.

A node is usually indicated by a dot in a circuit. If a short circuit  (a connecting wire) connects two nodes, the two nodes constitute a sin gle node. The circuit in Fig. 2.10 has three nodes a, b, and c. Notice that

a b

5 Ω

+– 2 Ω 3 Ω

10 V 2 A

c

Figure 2.10

Nodes, branches, and loops.

36 Chapter 2 Basic Laws

a

5 Ω

+–

10 V

b

2 Ω3 Ω 2 A c

the three points that form node b are connected by perfectly conducting  wires and therefore constitute a single point. The same is true of the four  points forming node c. We demonstrate that the circuit in Fig. 2.10 has  only three nodes by redrawing the circuit in Fig. 2.11. The two circuits in  Figs. 2.10 and 2.11 are identical. However, for the sake of clarity, nodes  b and c are spread out with perfect conductors as in Fig. 2.10.

A loop is any closed path in a circuit.

Figure 2.11

The three-node circuit of Fig. 2.10 is   redrawn.

Example 2.4

A loop is a closed path formed by starting at a node, passing through a  set of nodes, and returning to the starting node without passing through  any node more than once. A loop is said to be independent if it contains  at least one branch which is not a part of an y other independent loop.  Independent loops or paths result in independent sets of equations. It is possible to form an independent set of loops where one of the

loops does not contain such a branch. In Fig. 2.11, abca with the 2 Ω re sistor is independent. A second loop with the 3 Ω resistor and the current  source is independent. The third loop could be the one with the 2Ω resistor  in parallel with the 3Ω resistor. This does form an independent set of loops.

A network with b branches, n nodes, and l independent loops will  satisfy the fundamental theorem of network topology:

b = l + n − 1 (2.12)

As the next two definitions show, circuit topology is of great value to  the study of voltages and currents in an electric circuit.

Two or more elements are in series if they exclusively share a single node  and consequently carry the same current.

Two or more elements are in parallel if they are connected to the same  two nodes and consequently have the same voltage across them.

Elements are in series when the y are chain-connected or connected se - quentially, end to end. F or example, two elements are in series if they  share one common node and no other element is connected to that  common node. Elements in parallel are connected to the same pair of  terminals. Elements may be connected in a w ay that they are neither in  series nor in parallel. In the circuit shown in Fig. 2.10, the voltage source  and the 5- Ω resistor are in series because the same current will flow  through them. The 2-Ω resistor, the 3-Ω resistor, and the current source  are in parallel because they are connected to the same two nodes b and c and consequently have the same voltage across them. The 5-Ω and 2-Ω resistors are neither in series nor in parallel with each other.

Determine the number of branches and nodes in the circuit shown in  Fig. 2.12. Identify which elements are in series and which are in parallel.

Solution:

Since there are four elements in the circuit, the circuit has four branches:  10 V, 5 Ω, 6 Ω, and 2 A. The circuit has three nodes as identified in Fig. 2.13.  The 5-Ω resistor is in series with the 10-V voltage source because the same current would flow in both. The 6-Ω resistor is in parallel with the 2-A cur rent source because both are connected to the same nodes 2 and 3.

5 Ω

10 V 6 Ω 2 A +–

Figure 2.12

For Example 2.4.

2.4 Kirchhoff’s Laws 37 1 5 Ω 2

10 V 6 Ω 2 A +–

3

Figure 2.13

The three nodes in the circuit of

Fig. 2.12.

How many branches and nodes does the circuit in Fig. 2.14 have? Identify Practice Problem 2.4 the elements that are in series and in parallel.

Answer: Five branches and three nodes are identified in Fig. 2.15. The

1-Ω and 2-Ω resistors are in parallel. The 4-Ω resistor and 10-V source

are also in parallel.

1 2

5 Ω

1 Ω 2 Ω 10 V 4 Ω +–

Figure 2.14

For Practice Prob. 2.4.

2.4 Kirchhoff’s Laws

5 Ω

1 Ω 2 Ω +– 10 V 4 Ω

3

Figure 2.15

Answer for Practice Prob. 2.4.

Ohm’s law by itself is not sufficient to analyze circuits. However, when it

is coupled with Kirchhoff’s two laws, we have a sufficient, powerful set

of tools for analyzing a large variety of electric circuits. Kirchhoff’s laws

were first introduced in 1847 by the German physicist Gustav Robert

Kirchhoff (1824–1887). These laws are formally kno wn as Kirchhoff’s

current law (KCL) and Kirchhoff’s voltage law (KVL).

Kirchhoff’s first law is based on the la w of conservation of charge,

which requires that the algebraic sum of charges within a system cannot

change.

Kirchhoff’s current law (KCL) states that the algebraic sum of currents

entering a node (or a closed boundary) is zero.

Mathematically, KCL implies that

Nin = 0 (2.13)

∑

n=1

where N is the number of branches connected to the node and in is the nth

current entering (or lea ving) the node. By this la w, currents entering a

node may be regarded as positive, while currents leaving the node may

be taken as negative or vice versa.

38 Chapter 2 Basic Laws

Historical

Gustav Robert Kirchhoff (1824–1887), a German physicist, stated

two basic laws in 1847 concerning the relationship between the cur -

rents and voltages in an electrical network. Kirchhoff’s laws, along with

Ohm’s law, form the basis of circuit theory.

Born the son of a lawyer in Konigsberg, East Prussia, Kirchhoff

© Pixtal/age Fotostock RF

i1i5

i4

i2

i3

Figure 2.16

entered the University of Konigsberg at age 18 and later became a lectur er in Berlin. His collaborative work in spectroscopy with German chem ist Robert Bunsen led to the discovery of cesium in 1860 and rubidium

in 1861. Kirchhoff was also credited with the Kirchhoff law of radiation.  Thus, Kirchhoff is famous among engineers, chemists, and physicists.

To prove KCL, assume a set of currents i k (t) , k = 1, 2,…, flow into  a node. The algebraic sum of currents at the node is

iT (t) = i1(t) + i2(t) + i3(t) + ⋯ (2.14) Integrating both sides of Eq. (2.14) gives

qT (t) = q1(t) + q2(t) + q3(t) + ⋯ (2.15)

where qk (t) = ∫ ik (t) d t and qT (t) = ∫ iT (t) d t .But the law of conserva tion of electric charge requires that the algebraic sum of electric charges  at the node must not change; that is, the node stores no net charge. Thus,  qT (t) = 0 → iT (t) = 0, confirming the validity of KCL.

Currents at a node illustrating KCL. Closed boundary

Figure 2.17

Applying KCL to a closed boundary.

Two sources (or circuits in general) are  said to be equivalent if they have the  same i-v relationship at a pair of

terminals.

Consider the node in Fig. 2.16. Applying KCL gives

i1 + (−i2) + i3 + i4 + (−i5) = 0 (2.16)

since currents i1, i3, and i4 are entering the node, while currents i2 and i5 are leaving it. By rearranging the terms, we get

i1 + i3 + i4 = i2 + i5 (2.17)

Equation (2.17) is an alternative form of KCL:

The sum of the currents entering a node is equal to the sum of the cur rents leaving the node.

Note that KCL also applies to a closed boundary . This may be re - garded as a generalized case, because a node may be regarded as a closed  surface shrunk to a point. In tw o dimensions, a closed boundary is the  same as a closed path. As typically illustrated in the circuit of Fig. 2.17,  the total current entering the closed surface is equal to the total current  leaving the surface.

A simple application of KCL is combining current sources in paral lel. The combined current is the algebraic sum of the current supplied  by the indi vidual sources. F or example, the current sources sho wn in

2.4 Kirchhoff’s Laws 39

Fig. 2.18(a) can be combined as in Fig. 2.18(b). The combined or equiva

IT

lent current source can be found by applying KCL to node a.

a

IT +​I2 =​I1 +​I3

or

b

IT = I1 − I2 + I3 (2.18)

A circuit cannot contain two different currents, I1 and I2, in series, unless  I1 =I2; otherwise KCL will be violated.

Kirchhoff’s second law is based on the principle of conservation of  energy:

Kirchhoff’s voltage law (KVL) states that the algebraic sum of all voltages

I1 I2 I3

(a)

IT

a

IT = I1 – I2 + I3

b

(b)

around a closed path (or loop) is zero.

Expressed mathematically, KVL states that

Mvm = 0 (2.19)

∑

m=1

where M is the number of voltages in the loop (or the number of branches  in the loop) and vm is the mth voltage.

To illustrate KVL, consider the circuit in Fig. 2.19. The sign on  each voltage is the polarity of the terminal encountered first as we travel  around the loop. We can start with an y branch and go around the loop  either clockwise or counterclockwise. Suppose we start with the v olt age source and go clockwise around the loop as sho wn; then v oltages  would be −v1, +v2, +v3, +v4, and −v5, in that order. For example, as we  reach branch 3, the positive terminal is met first; hence, we have +v3. For  branch 4, we reach the ne gative terminal first; hence, −v4. Thus, KVL  yields

−v1 + v2 + v3 − v4 + v5 = 0 (2.20)

Rearranging terms gives

v2 + v3 + v5 = v1 + v4 (2.21)

which may be interpreted as

Sum of voltage drops = Sum of voltage rises (2.22)

This is an alternative form of KVL. Notice that if we had traveled coun terclockwise, the result w ould have been +v1, −v5, +v4, −v3, and −v2,  which is the same as before e xcept that the signs are re versed. Hence,  Eqs. (2.20) and (2.21) remain the same.

When voltage sources are connected in series, KVL can be applied  to obtain the total v oltage. The combined v oltage is the algebraic sum  of the v oltages of the indi vidual sources. F or example, for the v oltage  sources shown in Fig. 2.20(a), the combined or equivalent voltage source  in Fig. 2.20(b) is obtained by applying KVL.

−Vab + V1 + V2 − V3 = 0

Figure 2.18

Current sources in parallel: (a) original  circuit, (b) equivalent circuit.

KVL can be applied in two ways: by

taking either a clockwise or a counter

clockwise trip around the loop. Either  way, the algebraic sum of voltages

around the loop is zero.

+ – + –

v3 v2

v v1 4 +– +–

– +

v5

Figure 2.19

A single-loop circuit illustrating KVL.

40 Chapter 2 Basic Laws

or

Vab = V1 + V2 − V3 (2.23)

To avoid violating KVL, a circuit cannot contain tw o different voltages

V1 and V2 in parallel unless V1 =V2.

a

+

+–

V1

b

Figure 2.20

+– Vab

+–

–

(a)

V2 V3

a

b

(b)

+

Vab –

+–

VS = V1 + V2 – V3

Voltage sources in series: (a) original circuit, (b) equivalent circuit.

For the circuit in Fig. 2.21(a), find voltages v1 and v2 Example 2.5 .

2 Ω

v1 + –

–

2 Ω

v1 + –

–

20 V +– v2 3 Ω

20 V +– v2 3 Ω

+

(a)

Figure 2.21

For Example 2.5.

Solution:

i

(b)

+

To find v1 and v2 we apply Ohm’s law and Kirchhoff’s voltage law.  Assume that current i flows through the loop as shown in Fig. 2.21(b).  From Ohm’s law,

v1 = 2i, v2 = −3i (2.5.1)

Applying KVL around the loop gives

−20 + v1 − v2 = 0 (2.5.2)

Substituting Eq. (2.5.1) into Eq. (2.5.2), we obtain

−20 + 2i + 3i = 0 or 5i = 20 ⇒ i = 4 A

Substituting i in Eq. (2.5.1) finally gives

v1 = 8 V, v2 = −12 V

2.4 Kirchhoff’s Laws 41

Find v1 and v2 in the circuit of Fig. 2.22. Practice Problem 2.5

Answer: 16 V, −8 V.

4 Ω

+ – v1

–

32 V +– –8 V +

+ –

v2

2 Ω

Figure 2.22

For Practice Prob. 2.5.

Determine v Example 2.6 o and i in the circuit shown in Fig. 2.23(a).

i

4 Ω

2vo

+ –

4 Ω

2vo

+ –

12 V

+– +–

4 V

6 Ω

12 V

i

+– +–

4 V

6 Ω

vo + – + – vo

(a)

Figure 2.23

For Example 2.6. Solution:

(b)

We apply KVL around the loop as shown in Fig. 2.23(b). The result is

−12 + 4i + 2 vo − 4 + 6i = 0 (2.6.1)

Applying Ohm’s law to the 6-Ω resistor gives

vo = −6i (2.6.2)

Substituting Eq. (2.6.2) into Eq. (2.6.1) yields

−16 + 10i − 12i = 0 ⇒ i = −8 A

and vo = 48 V.

Find vx and vo in the circuit of Fig. 2.24. Practice Problem 2.6

Answer: 20 V, −10 V.

10 Ω

+ –

vx

70 V 2vx +– +–

5 Ω

vo + –

Figure 2.24

For Practice Prob. 2.6.

42 Chapter 2 Basic Laws

Example 2.7

a

+

Find current io and voltage vo in the circuit shown in Fig. 2.25.

Solution:

Applying KCL to node a, we obtain

io

3 + 0.5io = io ⇒ io = 6 A

0.5io 3 A

vo 4 Ω

–

Figure 2.25

For Example 2.7.

Practice Problem 2.7

io

+

io

For the 4-Ω resistor, Ohm’s law gives vo = 4io = 24 V

Find vo and io in the circuit of Fig. 2.26. Answer: 20 V, 10 A.

15 A 2 Ω 12 Ω vo

–

Figure 2.26

For Practice Prob. 2.7.

Example 2.8 Find currents and voltages in the circuit shown in Fig. 2.27(a).

8 Ω

+ –

i3 i1 a

8 Ω

+ –

i3 i1 a

v1 i2 +

+

v1 i2 +

+

30 V +–

v2 3 Ω v3 6 Ω

30 V +–

Loop 1 Loop 2

–

v2 3 Ω v3 6 Ω

–

Figure 2.27 For Example 2.8.

(a)

Solution:

–

(b)

–

We apply Ohm’s law and Kirchhoff’s laws. By Ohm’s law,

v1 = 8i1, v2 = 3i2, v3 = 6i3 (2.8.1)

Since the voltage and current of each resistor are related by Ohm’s  law as shown, we are really looking for three things: (v1, v2, v3) or (i1, i2, i3).  At node a, KCL gives

i1 − i2 − i3 = 0 (2.8.2)

Applying KVL to loop 1 as in Fig. 2.27(b),

−30 + v1 + v2 = 0

2.5 Series Resistors and Voltage Division 43

We express this in terms of i1 and i2 as in Eq. (2.8.1) to obtain

−30 + 8 i1 + 3 i2 = 0

or

i1 = (30 − 3 i2) ______

8 (2.8.3)

Applying KVL to loop 2,

−v2 + v3 = 0 ⇒ v3 = v2 (2.8.4)

as expected since the two resistors are in parallel. We express v1 and v2 in

terms of i1 and i2 as in Eq. (2.8.1). Equation (2.8.4) becomes

6 i3 = 3 i2 ⇒ i3 = i__22 (2.8.5)

Substituting Eqs. (2.8.3) and (2.8.5) into (2.8.2) gives

30 − 3 i _____2

8 − i2 − i__22 = 0

or i2 = 2 A. From the v alue of i2, we now use Eqs. (2.8.1) to (2.8.5) to

obtain

i1 = 3 A, i3 = 1 A, v1 = 24 V, v2 = 6 V, v3 = 6 V

Find the currents and voltages in the circuit shown in Fig. 2.28. Practice Problem 2.8

Answer: v1 = 6 V, v2 = 4 V, v3 = 10 V, i1 = 3 A, i2 = 500 mA, i3 = 2.5 A. 2 Ω

i3 i1

4 Ω

+ – + –

v1

+

i2

v3

+–

2.5 Series Resistors and Voltage Division

The need to combine resistors in series or in parallel occurs so frequently  that it warrants special attention. The process of combining the resistors  is facilitated by combining two of them at a time. With this in mind, con sider the single-loop circuit of Fig. 2.29. The two resistors are in series,  since the same current i flows in both of them. Applying Ohm’s law to  each of the resistors, we obtain

v1 = iR1 , v2 = iR2 (2.24)

If we apply KVL to the loop (mo ving in the clockwise direction), we

10 V 6 V +–

v2 8 Ω

–

Figure 2.28

For Practice Prob. 2.8.

have

i

a

R1

R2

−v + v1 + v2 = 0 (2.25)

+ – + –

Combining Eqs. (2.24) and (2.25), we get

v = v1 + v2 = i(R1 + R2) (2.26)

or

v1

v +–

b

v2

i = _____ v

R1 + R2 (2.27)

Figure 2.29

A single-loop circuit with two resistors  in series.

44 Chapter 2 Basic Laws

Notice that Eq. (2.26) can be written as

v = iReq (2.28)

i

Req

a

+ –

v

+–

v

implying that the two resistors can be replaced by an equivalent resistor  Req; that is,

Req = R1 + R2 (2.29)

Thus, Fig. 2.29 can be replaced by the equivalent circuit in Fig. 2.30. The

b

Figure 2.30

two circuits in Figs. 2.29 and 2.30 are equivalent because they exhibit the  same voltage-current relationships at the terminals a-b. An equivalent

Equivalent circuit of the Fig. 2.29 circuit.

Resistors in series behave as a single   resistor whose resistance is equal to  the sum of the resistances of the   individual resistors.

circuit such as the one in Fig. 2.30 is useful in simplifying the analysis  of a circuit. In general,

The equivalent resistance of any number of resistors connected in series  is the sum of the individual resistances.

For N resistors in series then,

NRn (2.30)

Req = R1 + R2 + ⋯ + RN = ∑n=1

To determine the voltage across each resistor in Fig. 2.29, we substi tute Eq. (2.26) into Eq. (2.24) and obtain

v1 = _____ R1

R1 + R2 v, v2 = _____ R2

R1 + R2 v (2.31)

Notice that the source v oltage v is divided among the resistors in direct  proportion to their resistances; the larger the resistance, the larger the  voltage drop. This is called the principle of voltage division, and the cir cuit in Fig. 2.29 is called a voltage divider. In general, if a voltage divider

has N resistors (R1, R2, … , RN) in series with the source voltage v, the nth  resistor (Rn) will have a voltage drop of

vn = __________ Rn R1 + R2 + ⋯ + RN v (2.32)

2.6 Parallel Resistors and Current Division

i

Node a

i1 i2

Consider the circuit in Fig. 2.31, where tw o resistors are connected  in  parallel and therefore ha ve the same v oltage across them. From  Ohm’s law,

v +– R1 R2

Node b

Figure 2.31

Two resistors in parallel.

v = i1R1 = i2R2

or

i1 = __vR1 , i2 = __vR2 (2.33)

Applying KCL at node a gives the total current i as

i = i1 + i2 (2.34)

Substituting Eq. (2.33) into Eq. (2.34), we get

i = __vR1 + __vR2 = v (__1R1 + __1R2 ) = ___vReq (2.35)

2.6 Parallel Resistors and Current Division 45

where Req is the equivalent resistance of the resistors in parallel:

___1Req = __1R1 + __1R2 (2.36)

or

R1 + R2

___1Req =​​______

R1R2

or

Req = _____ R1R2

R1 +​R2(2.37)

Thus,

The equivalent resistance of two parallel resistors is equal to the prod

uct of their resistances divided by their sum.

It must be emphasized that this applies only to tw o resistors in parallel.

From Eq. (2.37), if R1 = R2 then Req = R1/R2.

We can extend the result in Eq. (2.36) to the general case of a circuit

with N resistors in parallel. The equivalent resistance is

___1Req = __1R1 + __1R2 + ⋯ + ___1RN (2.38)

Note that Req is always smaller than the resistance of the smallest resistor

in the parallel combination. If R1 = R2 = ⋯​=​RN = R, then

Req = __RN (2.39)

For e xample, if four 100- Ω resistors are connected in parallel, their  equivalent resistance is 25 Ω.

It is often more convenient to use conductance rather than resistance  when dealing with resistors in parallel. From Eq. (2.38), the equi valent  conductance for N resistors in parallel is

Geq = G1 + G2 + G3 + ⋯ + GN (2.40)

where Geq = 1/Req, G1 = 1/R1, G2 = 1/R2, G3 = 1/ R3, … , GN = 1/ RN.  Equation (2.40) states:

The equivalent conductance of resistors connected in parallel is the  sum of their individual conductances.

This means that we may replace the circuit in Fig. 2.31 with that in  Fig. 2.32. Notice the similarity between Eqs. (2.30) and (2.40). The  equivalent conductance of parallel resistors is obtained the same w ay  as the equivalent resistance of series resistors. In the same manner , the  equivalent conductance of resistors in series is obtained just the same

Conductances in parallel behave as a

single conductance whose value is

equal to the sum of the individual

conductances.

i

a

v +– Req or Geq v

b

Figure 2.32

Equivalent circuit to Fig. 2.31.

46 Chapter 2 Basic Laws

way as the resistance of resistors in parallel. Thus, the equivalent con

ductance Geq of N resistors in series (such as shown in Fig. 2.29) is

___1Geq = __1G1 + __1G2 + __1G3 + ⋯ + ___1GN (2.41)

Given the total current i entering node a in Fig. 2.31, ho w do we

i

i1 = 0 i2 = i

obtain current i1 and i2? We know that the equi valent resistor has the  same voltage, or

R2 = 0

R1

(a)

i

v = iReq = _____ iR1 R2

R1 + R2 (2.42)

Combining Eqs. (2.33) and (2.42) results in

i1 = R2 i _____

R1 + R2 , i2 = R1 i _____

i1 = i i2 = 0

R1 + R2 (2.43)

R2 = ∞

R1

(b)

Figure 2.33

which shows that the total current i is shared by the resistors in in - verse proportion to their resistances. This is known as the principle  of current division, and the circuit in Fig. 2.31 is known as a current  divider. Notice that the lar ger current flows through the smaller re - sistance.

As an extreme case, suppose one of the resistors in Fig. 2.31 is zero,

(a) A shorted circuit, (b) an open circuit.

say R2 = 0; that is, R2 is a short circuit, as sho wn in Fig. 2.33(a). From  Eq. (2.43), R2 = 0 implies that i1 = 0, i2 = i. This means that the entire  current i bypasses R1 and flows through the short circuit R2 = 0, the path  of least resistance. Thus when a circuit is short circuited, as sho wn in  Fig. 2.33(a), two things should be kept in mind:

1. The equivalent resistance Req = 0. [See what happens when R2 = 0  in Eq. (2.37).]

2. The entire current flows through the short circuit.

As another e xtreme case, suppose R2 = ∞, that is, R2 is an open  circuit, as shown in Fig. 2.33(b). The current still flows through the path  of least resistance, R1. By taking the limit of Eq. (2.37) as R2 → ∞,we  obtain Req = R1 in this case.

If we divide both the numerator and denominator by R1R2, Eq. (2.43)  becomes

i1 = _____ G1

G1 + G2 i (2.44a)

i2 = _____ G2

G1 + G2 i (2.44b)

Thus, in general, if a current divider has N conductors (G1, G2, … , GN) in  parallel with the source current i, the nth conductor (Gn) will have current

in = __________ Gn G1 + G2 + ⋯ + GN i (2.45)

2.6 Parallel Resistors and Current Division 47

In general, it is often convenient and possible to combine resistors in

series and parallel and reduce a resisti ve network to a single equivalent

resistance Req. Such an equi valent resistance is the resistance between

the designated terminals of the netw ork and must e xhibit the same i-v

characteristics as the original network at the terminals.

Find Req for the circuit shown in Fig. 2.34. Example 2.9

Solution:

4 Ω

To get Req, we combine resistors in series and in parallel. The 6- Ω and  3-Ω resistors are in parallel, so their equivalent resistance is

1 Ω

2 Ω

6 Ω ∥ 3Ω = ____ 6 × 3

5 Ω Req

6 + 3 = 2 Ω

(The symbol ∥ is used to indicate a parallel combination.) Also, the 1-Ω and 5-Ω resistors are in series; hence their equivalent resistance is

1 Ω + 5 Ω = 6 Ω

Thus the circuit in Fig. 2.34 is reduced to that in Fig. 2.35(a). In  Fig. 2.35(a), we notice that the tw o 2-Ω resistors are in series, so the  equivalent resistance is

2 Ω + 2 Ω = 4 Ω

This 4-Ω resistor is now in parallel with the 6-Ω resistor in Fig. 2.35(a);  their equivalent resistance is

4 Ω ∥ 6 Ω = ____ 4 × 6

4 + 6 = 2.4 Ω

The circuit in Fig. 2.35(a) is now replaced with that in Fig. 2.35(b). In  Fig. 2.35(b), the three resistors are in series. Hence, the equivalent resis tance for the circuit is

8 Ω

Figure 2.34 For Example 2.9.

4 Ω

Req

8 Ω

Req

Figure 2.35

6 Ω 3 Ω

2 Ω

2 Ω

(a)

4 Ω

2.4 Ω

8 Ω

(b)

6 Ω

Req = 4 Ω + 2.4 Ω + 8 Ω = 14.4 Ω

Equivalent circuits for Example 2.9.

By combining the resistors in Fig. 2.36, find Req. Practice Problem 2.9

Answer: 11 Ω.

4 Ω

Req

3 Ω

4 Ω 8 Ω

6 Ω 12 Ω 10 Ω 6 Ω

Figure 2.36

For Practice Prob. 2.9.

48 Chapter 2 Basic Laws

Calculate the equivalent resistance Rab Example 2.10 in the circuit in Fig. 2.37. 10 Ω 1 Ω 1 Ω

a

Rab 3 Ω

c d 6 Ω

12 Ω

4 Ω 5 Ω

bb b

Figure 2.37

For Example 2.10.

Solution:

a

10 Ω 1 Ω c

The 3-Ω and 6-Ω resistors are in parallel because they are connected to  the same two nodes c and b. Their combined resistance is

d

3 Ω ∥ 6 Ω = ____ 3 × 6

3 + 6 = 2 Ω (2.10.1)

b

a b

b

10 Ω

2 Ω 3 Ω 6 Ω

bb

(a)

c

2 Ω 3 Ω b b

Similarly, the 12-Ω and 4-Ω resistors are in parallel since the y are con nected to the same two nodes d and b. Hence

12 Ω ∥ 4 Ω = ____ 12 × 4

12 + 4 = 3 Ω (2.10.2)

Also the 1-Ω and 5-Ω resistors are in series; hence, their equivalent re - sistance is

1 Ω + 5 Ω = 6 Ω (2.10.3)

With these three combinations, we can replace the circuit in Fig. 2.37 with  that in Fig. 2.38(a). In Fig. 2.38(a), 3-Ω in parallel with 6-Ω gives 2-Ω, as

(b)

Figure 2.38

calculated in Eq. (2.10.1). This 2-Ω equivalent resistance is now in series  with the 1-Ω resistance to give a combined resistance of 1 Ω +​2​Ω​=​3​Ω.

Equivalent circuits for Example 2.10.

Practice Problem 2.1020 Ω

Thus, we replace the circuit in Fig. 2.38(a) with that in Fig. 2.38(b). In  Fig. 2.38(b), we combine the 2-Ω and 3-Ω resistors in parallel to get

2 Ω ∥ 3 Ω = ____ 2 × 3

2 + 3 = 1.2 Ω

This 1.2-Ω resistor is in series with the 10-Ω resistor, so that Rab = 10 + 1.2 = 11.2 Ω

Find Rab for the circuit in Fig. 2.39.

Answer: 19 Ω.

a

16 Ω 5 Ω

Rab

18 Ω

1 Ω

9 Ω

20 Ω

b

Figure 2.39

For Practice Prob. 2.10.

2 Ω

2.6  Parallel Resistors and Current Division 49

Find the equivalent conductance Geq for the circuit in Fig. 2.40(a).

Solution:

The 8-S and 12-S resistors are in parallel, so their conductance is 8 S + 12 S = 20 S

This 20-S resistor is now in series with 5 S as shown in Fig. 2.40(b) so  that the combined conductance is

____ 20 × 5

20 + 5 ​= 4 S

This is in parallel with the 6-S resistor. Hence,

Geq = 6 + 4 = 10 S

We should note that the circuit in Fig. 2.40(a) is the same as that in  Fig. 2.40(c). While the resistors in Fig. 2.40(a) are expressed in siemens,  those in Fig. 2.40(c) are expressed in ohms. To show that the circuits are  the same, we find Req for the circuit in Fig. 2.40(c).

Req = __16 ​​​∥( __15 ​+ __18 ​∥ ___112) = __16 ​​∥ (__15 ​+ ___120 ​) = __16 ​∥ __14 ​​

= _16 × _1

_16 + _14= ___110 ​​Ω

____4

Geq = ___1Req ​​= 10 S

This is the same as we obtained previously.

Example 2.11

5 S

Geq

6 S 8 S 12 S

(a)

5 S

Geq

6 S 20 S

(b)

Ω15

Req

Ω16 Ω18 Ω112

(c)

Figure 2.40

For Example 2.11: (a) original circuit,  (b) its equivalent circuit, (c) same circuit as  in (a) but resistors are expressed in ohms.

Calculate Geq in the circuit of Fig. 2.41. Practice Problem 2.11 Answer: 8 S.

Geq

Figure 2.41

7 S

16 S

5 S 8 S

Find io and vo in the circuit shown in Fig. 2.42(a). Calculate the power  dissipated in the 3-Ω resistor.

Solution:

The 6-Ω and 3-Ω resistors are in parallel, so their combined resistance is 6 Ω ∥ 3 Ω = ____ 6 × 3

6 + 3 ​= 2 Ω

Thus, our circuit reduces to that shown in Fig. 2.42(b). Notice that vo is  not affected by the combination of the resistors because the resistors are

For Practice Prob. 2.11.

Example 2.12

50 Chapter 2 Basic Laws

12 V

4 Ω i io

a

+

6 Ω vo 3 Ω

+–

–

b

(a)

in parallel and therefore have the same voltage vo. From Fig. 2.42(b), we  can obtain vo in two ways. One way is to apply Ohm’s law to get

i = _____ 12

4 + 2 = 2 A

and hence, vo = 2i = 2 × 2 = 4 V. Another way is to apply voltage divi sion, since the 12 V in Fig. 2.42(b) is divided between the 4-Ω and 2-Ω resistors. Hence,

vo = _____ 2

4 Ω i

+

2 + 4 (12 V) = 4 V

a

Similarly, io can be obtained in two ways. One approach is to apply

12 V

+– vo 2 Ω –

b

(b)

Ohm’s law to the 3-Ω resistor in Fig. 2.42(a) now that we know vo; thus, vo = 3 io = 4 ⇒ io = __43 A

Another approach is to apply current division to the circuit in Fig. 2.42(a)

Figure 2.42

For Example 2.12: (a) original circuit,  (b) its equivalent circuit.

Practice Problem 2.12

now that we know i, by writing

io = _____ 6

6 + 3 i = __23 (2 A) =​__43 A

The power dissipated in the 3-Ω resistor is

po = voio = 4 (__43 ) = 5.333 W

Find v1 and v2 in the circuit sho wn in Fig. 2.43. Also calculate i1 and i2 and the power dissipated in the 12-Ω and 40-Ω resistors.

i1

12 Ω

+ –

v1

6 Ω

i2

+

Answer: v1 = 10 V, i1 = 833.3 mA, p1 = 8.333 W, v2 = 20 V, i2 =500 mA,  p2 = 10 W.

+– v2 40 Ω

30 V

Figure 2.43

For Practice Prob. 2.12.

10 Ω

–

Example 2.13

For the circuit sho wn in Fig. 2.44(a), determine: (a) the v oltage vo,  (b) the power supplied by the current source, (c) the power absorbed by  each resistor.

Solution:

(a) The 6-k Ω and 12-k Ω resistors are in series so that their combined  value is 6 + 12 = 18 kΩ. Thus the circuit in Fig. 2.44(a) reduces to that  shown in Fig. 2.44(b). We now apply the current division technique to

find i1 and i2.

i1 =​​_____________ 18,000 9,000 + 18,000 (30 mA) = 20 mA

i2 = _____________ 9,000 9,000 + 18,000 (30 mA) = 10 mA

2.7 Wye-Delta Transformations 51

Notice that the voltage across the 9-kΩ and 18-kΩ resistors is the same,

6 kΩ

and vo = 9,000i1 = 18,000i2 = 180 V, as expected.

(b) Power supplied by the source is

+

po = voio = 180 (30) mW = 5.4 W

(c) Power absorbed by the 12-kΩ resistor is

p = iv = i2 (i2 R) = i 22 R = (10 ×​10−3)2 (12,000) =​1.2 W

30 mA vo 9 kΩ –

(a)

io i2

12 kΩ

Power absorbed by the 6-kΩ resistor is

i1

+

p = i22 R = (10 × 10−3)2 (6,000) = 0.6 W

Power absorbed by the 9-kΩ resistor is

p = v2

__oR = (180)2 _____

9,000 = 3.6 W

30 mA vo 9 kΩ –

(b)

Figure 2.44

18 kΩ

or

p = voi1 = 180(20) mW = 3.6 W

Notice that the power supplied (5.4 W) equals the power absorbed (1.2 + 0.6 + 3.6 = 5.4 W). This is one way of checking results.

For Example 2.13: (a) original circuit,  (b) its equivalent circuit.

For the circuit shown in Fig. 2.45, find: (a) v1 and v2, (b) the power dis- Practice Problem 2.13 sipated in the 3-k Ω and 20-kΩ resistors, and (c) the power supplied by

the current source.

3 kΩ

+

v1

+

v2

9 kΩ 30 mA 15 kΩ 60 kΩ

–

–

Figure 2.45

For Practice Prob. 2.13.

Answer: (a) 135 V, 180 V, (b) 2.025 W, 540 mW, (c) 5.4 W.

2.7 Wye-Delta Transformations

Situations often arise in circuit analysis when the resistors are neither  in parallel nor in series. For example, consider the bridge circuit in  Fig. 2.46. How do we combine resistors R1 through R6 when the resistors  are neither in series nor in parallel? Man y circuits of the type sho wn in  Fig. 2.46 can be simplified by using three-terminal equivalent networks.

vs +–

R1

R4

R2

R3

These are the wye (Y) or tee (T) netw ork shown in Fig. 2.47 and the  delta (Δ) or pi (Π) network shown in Fig. 2.48. These networks occur by  themselves or as part of a lar ger network. They are used in three-phase

R6

Figure 2.46

R5

networks, electrical filters, and matching networks. Our main interest

The bridge network.

52 Chapter 2 Basic Laws

1 3

R1 R2

R3

2 4

(a)

Figure 2.47

R1 R2

1 3 R3

2 4 (b)

Rc

1 3

Rb Ra

2 4

(a)

Rc

1 3

Ra Rb

2 4

(b)

Figure 2.48

Two forms of the same network: (a) Δ,  (b) Π.

Two forms of the same network: (a) Y, (b) T.

here is in how to identify them when they occur as part of a network and  how to apply wye-delta transformation in the analysis of that network.

Delta to Wye Conversion

Suppose it is more convenient to work with a wye network in a place  where the circuit contains a delta configuration. We superimpose a wye  network on the existing delta network and find the equivalent resistances  in the wye network. To obtain the equivalent resistances in the wye net

work, we compare the tw o networks and mak e sure that the resistance  between each pair of nodes in the Δ (or Π) network is the same as the  resistance between the same pair of nodes in the Y (or T) network. For  terminals 1 and 2 in Figs. 2.47 and 2.48, for example,

R12 (Y) = R1 + R3 (2.46)

R12(Δ)​=​Rb∥​(Ra​+​Rc)

Setting R12(Y) = R12(Δ) gives

R12 = R1 + R3 = Rb (Ra + Rc) _________

Ra + Rb + Rc (2.47a)

Similarly,

R13 = R1 + R2 = Rc (Ra + Rb) _________

Ra + Rb + Rc (2.47b)

R34 = R2 + R3 = Ra (Rb + Rc) _________

Ra + Rb + Rc (2.47c)

Subtracting Eq. (2.47c) from Eq. (2.47a), we get

R1 − R2 = Rc (Rb − Ra) _________

Ra + Rb + Rc (2.48)

Adding Eqs. (2.47b) and (2.48) gives

R1 = Rb ________ Rc

Ra + Rb + Rc(2.49)

and subtracting Eq. (2.48) from Eq. (2.47b) yields

R2 = Rc ________ Ra

Ra + Rb + Rc (2.50)

Subtracting Eq. (2.49) from Eq. (2.47a), we obtain

R3 = Ra ________ Rb

Ra + Rb + Rc(2.51)

2.7 Wye-Delta Transformations 53

We do not need to memorize Eqs. (2.49) to (2.51). To transform a ∆  a

network to Y, we create an extra node n as shown in Fig. 2.49 and follow  this conversion rule:

Each resistor in the Y network is the product of the resistors in the two  adjacent Δ branches, divided by the sum of the three Δ resistors.

One can follow this rule and obtain Eqs. (2.49) to (2.51) from Fig. 2.49.

Wye to Delta Conversion

To obtain the conversion formulas for transforming a wye network to an

Rc

b

R1 R2

n

Rb Ra R3

c

equivalent delta network, we note from Eqs. (2.49) to (2.51) that R1 R2 + R2 R3 + R3 R1 = Ra Rb Rc (Ra + Rb + Rc) _________________ (Ra + Rb + Rc)2

(2.52) =​ Ra Rb _______ Rc

Ra + Rb + Rc

Dividing Eq. (2.52) by each of Eqs. (2.49) to (2.51) leads to the follo w ing equations:

Ra = R1 R2 + R2R3 + R3R1 _________________ R1 (2.53)

Rb = R1 R2 + R2R3 + R3R1 _________________ R2 (2.54)

Rc = R1 R2 + R2R3 + R3R1 _________________ R3 (2.55)

From Eqs. (2.53) to (2.55) and Fig. 2.49, the con version rule for Y to Δ is as follows:

Each resistor in the Δ network is the sum of all possible products of Y  resistors taken two at a time, divided by the opposite Y resistor.

The Y and Δ networks are said to be balanced when

R1 = R2 = R3 = RY , Ra = Rb = Rc = R∆ (2.56) Under these conditions, conversion formulas become

RΔ

3 or RΔ = 3 RY (2.57)

RY = ___

One may w onder wh y RY is less than RΔ. Well, we notice that the  Y-connection is like a “series” connection while the Δ-connection is like  a “parallel” connection.

Note that in making the transformation, we do not take anything out  of the circuit or put in anything new. We are merely substituting different  but mathematically equivalent three-terminal network patterns to create  a circuit in which resistors are either in series or in parallel, allo wing us  to calculate Req if necessary.

Figure 2.49

Superposition of Y and Δ networks as an  aid in transforming one to the other.

54 Chapter 2 Basic Laws

Example 2.14 Convert the Δ network in Fig. 2.50(a) to an equivalent Y network. Rc

a b 25 Ω

a b

5 Ω

7.5 Ω

10 Ω 15 Ω

Rb Ra

R1 R2

R3

c

(a)

Figure 2.50

3 Ω

c

(b)

For Example 2.14: (a) original Δ network, (b) Y equivalent network.

Solution:

Using Eqs. (2.49) to (2.51), we obtain

Ra + Rb + Rc = ________ 10 × 25 15 + 10 + 25 = ___ 250

R1 = Rb _______ Rc

R2 = Rc _______ Ra

Ra + Rb + Rc = _____ 25 × 15

50 = 7.5 Ω

R3 = Ra _______ Rb

Ra + Rb + Rc = _____ 15 × 10

50 =3 Ω

50 = 5 Ω

The equivalent Y network is shown in Fig. 2.50(b).

Practice Problem 2.14 Transform the wye network in Fig. 2.51 to a delta network.

R1

R2

Answer: Ra = 140 Ω, Rb = 70 Ω, Rc = 35 Ω.

a b

10 Ω

Figure 2.51

20 Ω

R3 40 Ωc

For Practice Prob. 2.14. Example 2.15

Obtain the equivalent resistance Rab for the circuit in Fig. 2.52 and use  it to find current i.

Solution:

1. Define. The problem is clearly defined. Please note, this part nor mally will deservedly take much more time.

2. Present. Clearly, when we remove the voltage source, we end up with  a purely resistive circuit. Since it is composed of deltas and wyes, we  have a more comple x process of combining the elements together .

2.7 Wye-Delta Transformations 55

We can use wye-delta transformations as one approach to find a so lution. It is useful to locate the wyes (there are tw o of them, one at  n and the other at c) and the deltas (there are three: can, abn, cnb).

3. Alternative. There are different approaches that can be used to solve  this problem. Since the focus of Sec. 2.7 is the wye-delta transfor - mation, this should be the technique to use. Another approach would

+–

a a i

12.5 Ω

5 Ω

10 Ω

be to solve for the equi valent resistance by injecting one amp into

120 V c n

30 Ω

the circuit and finding the voltage between a and b; we will learn  about this approach in Chap. 4.

The approach we can apply here as a check w ould be to use  a wye-delta transformation as the first solution to the problem.  Later we can check the solution by starting with a delta-wye  transformation.

4. Attempt. In this circuit, there are two Y networks and three Δ networks.  Transforming just one of these will simplify the circuit. If we convert the  Y network comprising the 5-Ω, 10-Ω, and 20-Ω resistors, we may select

R1 = 10 Ω, R2 = 20 Ω, R3 = 5 Ω

Thus from Eqs. (2.53) to (2.55) we have

Ra = R1 R2 + R2 R3 + R3 R1 _____________ R1 = 10 × 20 + 20 × 5 + 5 × 10 ______________ 10  = ___ 350

10 = 35 Ω

Rb = R1 R2 + R2 R3 + R3 R1 _____________ R2 = ___ 350

20 = 17.5 Ω

Rc = R1 R2 + R2 R3 + R3 R1 _____________ R3 = ___ 3505 =​70 Ω

With the Y converted to Δ, the equivalent circuit (with the volt age source removed for now) is shown in Fig. 2.53(a). Combining  the three pairs of resistors in parallel, we obtain

70∥30 = _____ 70 × 30

70 + 30 = 21 Ω

12.5 ∥ 17.5 = ________ 12.5 × 17.5

12.5 + 17.5 = 7.292 Ω

15∥35 = _____ 15 × 35

15 + 35 = 10.5 Ω

a

a

15 Ω

b b

Figure 2.52

For Example 2.15.

4.545 Ω

d

20 Ω

12.5 Ω

17.5 Ω

a

70 Ω 30 Ω

7.292 Ω

2.273 Ω 1.8182 Ω c n

30 Ω

15 Ω

b

35 Ω (a)

b

21 Ω

10.5 Ω

(b)

b

15 Ω

(c)

20 Ω

Figure 2.53

Equivalent circuits to Fig. 2.52, with the voltage source removed.

56 Chapter 2 Basic Laws

so that the equivalent circuit is shown in Fig. 2.53(b). Hence, we find

Rab = (7.292 +​10.5) ∥ 21 = ________ 17.792 × 21

17.792 + 21 = 9.632 Ω

Then

i = ___ vs

Rab = ____ 120

9.632 = 12.458 A

We observe that we have successfully solved the problem. Now

we must evaluate the solution.

5. Evaluate. Now we must determine if the answer is correct and then

evaluate the final solution.

It is relatively easy to check the answer; we do this by solving

the problem starting with a delta-wye transformation. Let us trans -

form the delta, can, into a wye.

Let Rc = 10 Ω, Ra = 5 Ω, and Rn = 12.5 Ω. This will lead to (let

d represent the middle of the wye):

R a + R c + R n = ________ 10 × 12.5 5 + 10 +​12.5 = 4.545 Ω

R ad = R c R _______ n

R cd =  R a R ____n

27.5 = ______ 5 × 12.5

27.5 = 2.273 Ω

R nd =  R a R ____c

27.5 = ____ 5 × 10

27.5 = 1.8182 Ω

This now leads to the circuit sho wn in Figure 2.53(c). Looking at

the resistance between d and b, we have two series combination in

parallel, giving us

R db = (2.273 + 15)(1.8182 + 20) ________________ 2.273 + 15 + 1.8182 + 20 = ____ 376.9

39.09 = 9.642 Ω

This is in series with the 4.545-Ω resistor, both of which are in par

allel with the 30-Ω resistor. This then gives us the equivalent resis

tance of the circuit.

R ab = (9.642 + 4.545) 30 ____________ 9.642 + 4.545 + 30 = ____ 425.6

44.19 = 9.631 Ω

This now leads to

i = v ___s

R ab = ____ 120

9.631 = 12.46 A

We note that using tw o variations on the wye-delta transformation

leads to the same results. This represents a very good check.

6. Satisfactory? Since we have found the desired answer by deter -

mining the equi valent resistance of the circuit first and the an

swer checks, then we clearly ha ve a satisfactory solution. This

represents what can be presented to the indi vidual assigning the

problem.

For the bridge network in Fig. 2.54, find Rab and i.

2.8 Applications 57 Practice Problem 2.15

Answer: 60 Ω, 4 A.

2.8 Applications

6 Ω

48 Ω

240 V

60 Ω

i

+–

a

40 Ω

20 Ω 100 Ω

Resistors are often used to model de vices that con vert electrical ener - gy into heat or other forms of energy. Such devices include conducting  wire, light bulbs, electric heaters, stoves, ovens, and loudspeakers. In this  section, we will consider two real-life problems that apply the concepts

developed in this chapter: electrical lighting systems and design of dc  meters.

2.8.1 Lighting Systems

Lighting systems, such as in a house or on a Christmas tree, often con sist of N lamps connected either in parallel or in series, as sho wn in  Fig.  2.55. Each lamp is modeled as a resistor . Assuming that all the  lamps are identical and Vo is the power-line voltage, the voltage across

each lamp is Vo for the parallel connection and Vo /N for the series con nection. The series connection is easy to manufa cture but is seldom  used in practice, for at least two reasons. First, it is less reliable; when

a lamp fa ils, all the lamps go out. Second, it is harder to maintain;  when a lamp is bad, one must test all the lamps one by one to detect  the faulty one.

Historical

Thomas Alva Edison (1847–1931) was perhaps the greatest American  inventor. He patented 1093 inventions, including such history-making  inventions as the incandescent electric bulb, the phonograph, and the  first commercial motion pictures.

Born in Milan, Ohio, the youngest of seven children, Edison re ceived only three months of formal education because he hated school.  He was home-schooled by his mother and quickly began to read on  his own. In 1868, Edison read one of Faraday’s books and found his  calling. He moved to Menlo Park, New Jersey, in 1876, where he man - aged a well-staffed research laboratory. Most of his inventions came  out of this laboratory. His laboratory served as a model for modern re search  organ iza tions. Because of his diverse interests and the over - whelming number of his inventions and patents, Edison began to estab - lish manufacturing companies for making the devices he invented. He  designed the first electric power station to supply electric light. Formal  electrical engineering education began in the mid-1880s with Edison as  a role model and leader.

b

Figure 2.54

For Practice Prob. 2.15.

So far, we have assumed that connect ing wires are perfect conductors (i.e.,

conductors of zero resistance). In real  physical systems, however, the

resistance of the connecting wire may  be appreciably large, and the

modeling of the system must include

that resistance.

Library of Congress

58 Chapter 2 Basic Laws

+

Vo

–

12

+

Vo

–

Power

plug

Figure 2.55

1 2 3 N (a) Lamp

3

N

(b)

Example 2.16

(a) Parallel connection of light bulbs, (b) series connection of light bulbs.

Three light bulbs are connected to a 9-V battery as shown in Fig. 2.56(a).  Calculate: (a) the total current supplied by the battery , (b) the current  through each bulb, (c) the resistance of each bulb.

9 V

Figure 2.56

15 W

20 W

10 W

(a)

I

9 V

+

V2

–

+

V3

–

(b)

I1

I2

R2 R3

+

V1 R1 –

(a) Lighting system with three bulbs, (b) resistive circuit equivalent model.

Solution:

(a) The total po wer supplied by the battery is equal to the total po wer  absorbed by the bulbs; that is,

p = 15 + 10 + 20 = 45 W

Since p = V I, then the total current supplied by the battery is

I = p__V = __459 = 5 A

(b) The bulbs can be modeled as resistors as shown in Fig. 2.56(b). Since  R1 (20-W bulb) is in parallel with the battery as well as the series com - bination of R2 and R3,

V1 = V2 + V3 = 9 V

The current through R1 is

p1

V1 = __209 = 2.222 A

I1 = __

2.8 Applications 59

By KCL, the current through the series combination of R2 and R3 is

I2 = I − I1 = 5 − 2.222 = 2.778 A

(c) Since p = I2R,

p1

I 21 = _____ 20

R1 = ___ p2

2.2222  = 4.05 Ω

I 22 = _____ 15

R2 = ___ p3

2.7772  = 1.945 Ω​

I 23 = _____ 10

R3 = ___

2.7772  = 1.297 Ω​

Refer to Fig. 2.55 and assume there are six light b ulbs that can be con - nected in parallel and six dif ferent light bulbs that can be connected in  series. In either case, each light bulb is to operate at 40 W. If the voltage  at the plug is 115 V for the parallel and series connections, calculate the  current through and the voltage across each bulb for both cases.

Answer: 115 V and 347.8 mA (parallel), 19.167 V and 2.087 A (series).

2.8.2 Design of DC Meters

By their nature, resistors are used to control the flow of current. We take

Practice Problem 2.16

a

Max

b

advantage of this property in se veral applications, such as in a poten -

+–

Vin

+

tiometer (Fig. 2.57). The word potentiometer, derived from the w ords

Vout

potential and meter, implies that potential can be metered out. The po

Min

–

tentiometer (or pot for short) is a three-terminal de vice that operates on

c

the principle of v oltage division. It is essentially an adjustable v oltage

divider. As a voltage regulator, it is used as a volume or level control on  radios, TVs, and other devices. In Fig. 2.57,

Rbc

Vout = Vbc = ___

Rac Vin (2.58)

where Rac = Rab + Rbc. Thus, Vout decreases or increases as the sliding  contact of the pot moves toward c or a, respectively.

Another application where resistors are used to control current flow  is in the analog dc meters—the ammeter, voltmeter, and ohmmeter, which  measure current, voltage, and resistance, respectively. Each of these me ters employs the d’Arsonval meter movement, shown in Fig. 2.58. The  movement consists essentially of a movable iron-core coil mounted on  a pivot between the poles of a permanent magnet. When current flows  through the coil, it creates a torque which causes the pointer to deflect.  The amount of current through the coil determines the deflection of the  pointer, which is registered on a scale attached to the meter movement.  For example, if the meter movement is rated 1 mA, 50 Ω, it w ould take  1 mA to cause a full-scale deflection of the meter movement. By intro

ducing additional circuitry to the d’Arsonval meter mo vement, an am - meter, voltmeter, or ohmmeter can be constructed.

Consider Fig. 2.59, where an analog v oltmeter and ammeter are con - nected to an element. The voltmeter measures the voltage across a load and

Figure 2.57

The potentiometer controlling potential  levels.

An instrument capable of measuring  voltage, current, and resistance is

called a multimeter or a volt-ohm

meter (VOM).

A load is a component that is receiving  energy (an energy sink), as opposed

to a generator supplying energy (an

energy source). More about loading

will be discussed in Section 4.9.1.

60 Chapter 2 Basic Laws

pointer

N

scale

spring

S

permanent magnet

spring

Figure 2.58

rotating coil

stationary iron core

Ammeter A

+

A d’Arsonval meter movement.

I

is therefore connected in parallel with the element. As shown in Fig. 2.60(a),  the voltmeter consists of a d’Arson val movement in series with a resistor

Voltmeter

Figure 2.59

V

–

V

Element

whose resistance Rm is deliberately made very large (theoretically, infinite),  to minimize the current drawn from the circuit. To extend the range of volt age that the meter can measure, series multiplier resistors are often connected  with the voltmeters, as shown in Fig. 2.60(b). The multiple-range voltmeter  in Fig. 2.60(b) can measure v oltage from 0 to 1 V, 0 to 10 V, or 0 to 100 V,

Connection of a voltmeter and an  ammeter to an element.

depending on whether the switch is connected to R1, R2, or R3, respectively. Let us calculate the multiplier resistor Rn for the single-range voltmeter  in Fig. 2.60(a), or Rn =​ R1, R2, or R3 for the multiple-range voltmeter in  Fig. 2.60(b). We need to determine the value of Rn to be connected in series  with the internal resistance Rm of the voltmeter. In any design, we consider  the worst-case condition. In this case, the worst case occurs when the full scale current Ifs =Im flows through the meter. This should also correspond

+

Probes V –

R1

R2

Multiplier Rn

Im

(a)

1 V

10 V

Meter

Rm

Switch

Meter

+

Probes V

–

Figure 2.60

R3

Im

(b)

100 V

Rm

Voltmeters: (a) single-range type, (b) multiple-range type.

2.8 Applications 61

to the maximum v oltage reading or the full-scale v oltage Vfs. Since the  multiplier resistance Rn is in series with the internal resistance Rm,

Vfs = I fs (Rn +​Rm) (2.59)

From this, we obtain

Vfs

Rn = ___

Ifs − Rm (2.60)

Similarly, the ammeter measures the current through the load and is  connected in series with it. As shown in Fig. 2.61(a), the ammeter consists  of a d’Arsonval movement in parallel with a resistor whose resistance Rm is deliberately made very small (theoretically, zero) to minimize the v olt age drop across it. To allow multiple ranges, shunt resistors are often con nected in parallel with Rm as sho wn in Fig. 2.61(b). The shunt resistors

I Rn n

Meter

Im

Rm

I

Probes

(a)

R1

10 mA

allow the meter to measure in the range 0–10 mA, 0–100 mA, or 0–1 A,  R2

depending on whether the switch is connected to R1, R2, or R3, respectively. Now our objective is to obtain the multiplier shunt Rn for the single

R3

range ammeter in Fig. 2.61(a), or Rn =​R1, R2, or R3 for the multiple-range  ammeter in Fig. 2.61(b). We notice that Rm and Rn are in parallel and that  at full-scale reading I = Ifs = Im + In, where In is the current through the  shunt resistor Rn. Applying the current division principle yields

Im

Im = _____ Rn

Rn + Rm Ifs

I

or

Rn = _____ Im

Ifs − Im Rm (2.61)

100 mA

Meter

Rm

Probes (b)

Switch 1 A

The resistance Rx of a linear resistor can be measured in tw o ways.  An indirect way is to measure the current I that flows through it by con necting an ammeter in series with it and the v oltage V across it by con - necting a voltmeter in parallel with it, as shown in Fig. 2.62(a). Then

Rx = __VI (2.62)

The direct method of measuring resistance is to use an ohmmeter .  An ohmmeter consists basically of a d’Arson val movement, a v ariable

Figure 2.61

Ammeters: (a) single-range type,  (b) multiple-range type.

A

I

+

resistor or potentiometer, and a battery, as shown in Fig. 2.62(b). Apply ing KVL to the circuit in Fig. 2.62(b) gives

E = (R + Rm + Rx)Im

or

Rx V –

(a)

V

Rx = __EIm − (R +​Rm) (2.63)

Im

The resistor R is selected such that the meter gives a full-scale deflection;  that is, Im = Ifs when Rx = 0. This implies that

Rm

Ohmmeter R

E = (R + Rm) Ifs (2.64)

Substituting Eq. (2.64) into Eq. (2.63) leads to

Rx =​( I__fs

Im −1) (R + Rm) (2.65)

As mentioned, the types of meters we ha ve discussed are kno wn as  analog meters and are based on the d’Arsonval meter movement. Another  type of meter , called a digital meter, is based on acti ve circuit elements

E Rx

(b)

Figure 2.62

Two ways of measuring resistance:

(a) using an ammeter and a voltmeter,

(b) using an ohmmeter.

62 Chapter 2 Basic Laws

Historical

Samuel F. B. Morse (1791–1872), an American painter, invented

the telegraph, the first practical, commercialized application of

electricity.

Morse was born in Charlestown, Massachusetts, and studied at Yale

and the Royal Academy of Arts in London to become an artist. In the

1830s, he became intrigued with developing a telegraph. He had a working

model by 1836 and applied for a patent in 1838. The U.S. Senate appro -

priated funds for Morse to construct a telegraph line between Baltimore

and Washington, D.C. On May 24, 1844, he sent the famous first mes

sage: “What hath God wrought!” Morse also developed a code of dots and

dashes for letters and numbers, for sending messages on the telegraph. The

development of the telegraph led to the invention of the telephone.

Library of Congress

such as op amps. For example, a digital multimeter displays measurements

of dc or ac voltage, current, and resistance as discrete numbers, instead of

using a pointer deflection on a continuous scale as in an analog multimeter.

Digital meters are what you would most likely use in a modern lab. How

ever, the design of digital meters is beyond the scope of this book.

Example 2.17 Following the v oltmeter setup of Fig. 2.60, design a v oltmeter for the  following multiple ranges:

(a) 0–1 V (b) 0–5 V (c) 0–50 V (d) 0–100 V

Assume that the internal resistance Rm = 2 kΩ and the full-scale current

Ifs =​100​μA.

Solution:

We apply Eq. (2.60) and assume that R1, R2, R3, and R4 correspond with

ranges 0–1 V, 0–5 V, 0–50 V, and 0–100 V, respectively.

(a) For range 0–1 V,

R1 = _______ 1

100 × 10 −6 − 2000 = 10,000 − 2000 = 8 kΩ

(b) For range 0–5 V,

R2 = _______ 5

100 × 10 −6 − 2000 = 50,000 − 2000 = 48 kΩ

(c) For range 0–50 V,

R3 = _______ 50

100 × 10 −6 − 2000 = 500,000 − 2000 = 498 kΩ

(d) For range 0–100 V,

R4 = _______ 100 V

100 × 10 −6 − 2000 = 1,000,000 − 2000 = 998 kΩ

Note that the ratio of the total resistance (Rn + Rm) to the full-scale volt

age Vfs is constant and equal to 1/Ifs for the four ranges. This ratio (given

in ohms per v olt, or Ω​/ V) is known as the sensitivity of the v oltmeter.

The larger the sensitivity, the better the voltmeter.