SOAL ELEKTRONIKA ANALOG

BAB 3 Persimpangan Bipolar Transistor

Nama	:Mita Miranda

NIM	: 211331016

Kelas 	: 1A

- What names are applied to the two types of BJT transistors? Sketch the basic construction of each and label the various minority and majority carriers in each. Draw the graphic symbol next to each. Is any of this information altered by changing from a silicon to a germanium base?

(Nama apa yang diterapkan pada kedua jenis transistor BJT tersebut? Sketsa konstruksi dasar dari masing-masing dan beri label berbagai pembawa minoritas dan mayoritas di masing-masing. Gambar simbol grafik selanjutnya untuk masing-masing. Apakah ada dari informasi ini yang diubah dengan mengubah dari silikon ke basis germanium? )

Jawab:

Kedua jenis Bipolar Junction Transistor (BITS) adalah,

• Transistor PNP

• Transistor NPN Pada transistor PNP, semikonduktor tipe-n ditempatkan di antara dua semikonduktor tipe-p pada transistor NPN, semikonduktor tipe-p ditempatkan di antara  dua semikonduktor tipe n

- What is the major difference between a bipolar and a unipolar device?

(Apa perbedaan utama antara perangkat bipolar dan unipolar?)

Jawab:

Transistor bipolar menggunakan hole dan elektron pada proses injeksi atau perubahan aliran, sedangkan perangkat unipolar menggunakan elektron atau hole, tetapi tidak keduanya dalam proses perubahan.

- How must the two transistor junctions be biased for proper transistor amplifier operation?

(Bagaimana dua persimpangan transistor harus bias untuk operasi penguat transistor yang tepat?)

Jawab:

Bias maju dan bias mundur

- What is the source of the leakage current in a transistor?

(Apa sumber arus bocor pada transistor?)

Jawab:

arus bocor Ie0 adalah arus pembawa minoritas dalam kumpulan

- Sketch a figure similar to Fig. 3.3 for the forward-biased junction of an npn transistor. Describe the resulting carrier motion.

(Buat sketsa gambar yang mirip dengan Gambar 3.3 untuk sambungan bias maju dari transistor npn. Menggambarkan gerakan pembawa yang dihasilkan.)

Jawab:

- Sketch a figure similar to Fig. 3.4 for the reverse-biased junction of an npn transistor. Describe the resulting carrier motion.

(Buat sketsa gambar yang mirip dengan Gambar 3.4 untuk sambungan bias mundur dari transistor npn. Menggambarkan gerakan pembawa yang dihasilkan.)

Jawab:

- Sketch a figure similar to Fig. 3.5 for the majority- and minority-carrier flow of an npn transistor. Describe the resulting carrier motion.

(Buat sketsa gambar yang mirip dengan Gambar 3.5 untuk aliran pembawa mayoritas dan minoritas dari transistor npn. Jelaskan gerak pembawa yang dihasilkan.)

Jawab:

- Which of the transistor currents is always the largest? Which is always the smallest? Which two currents are relatively close in magnitude?

(Manakah dari arus transistor yang selalu terbesar? Manakah yang selalu paling kecil? Yang

dua arus yang besarnya relatif dekat?)

Jawab:

IE yang terpajang

IB yang terpendek

IE . IB

- If the emitter current of a transistor is 8 mA and IB is 1/100 of IC, determine the levels of IC and IB.

(Jika arus emitor transistor adalah 8 mA dan IB adalah 1/100 IC, tentukan level IC

dan IB.)

Jawab:

- From memory, sketch the transistor symbol for a pnp and an npn transistor, and then insert the conventional flow direction for each current.

(Dari memori, buat sketsa simbol transistor untuk transistor pnp dan transistor npn, lalu masukkan arah aliran konvensional untuk setiap arus.)

Jawab:

- Using the characteristics of Fig. 3.7, determine VBE at IE  5 mA for VCB  1, 10, and 20 V. Is it reasonable to assume on an approximate basis that VCB has only a slight effect on the relationship between VBE and IE?

- (a) Determine the average ac resistance for the characteristics of Fig. 3.10b.

  - For networks in which the magnitude of the resistive elements is typically in kilohms, isthe approximation of Fig. 3.10c a valid one [based on the results of part (a)]?

- (a) Using the characteristics of Fig. 3.8, determine the resulting collector current if IE  4.5 mA and VCB  4 V.

  - Repeat part (a) for IE  4.5 mA and VCB  16 V.

  - How have the changes in VCB affected the resulting level of IC?

  - On an approximate basis, how are IE and IC related based on the results above?

- (a) Using the characteristics of Figs. 3.7 and 3.8, determine IC if VCB  10 V and VBE  800 mV.

  - Determine VBE if IC  5 mA and VCB  10 V.

  - Repeat part (b) using the characteristics of Fig. 3.10b.

  - Repeat part (b) using the characteristics of Fig. 3.10c.

  - Compare the solutions for VBE for parts (b), (c), and (d). Can the difference be ignored if voltage levels greater than a few volts are typically encountered?

- (a) Given an dc of 0.998, determine IC if IE  4 mA. (b) Determine dc if IE  2.8 mA and IB  20 A. (c) Find IE if IB  40 A and dc is 0.98.

- From memory, and memory only, sketch the common-base BJT transistor configuration (for npn and pnp) and indicate the polarity of the applied bias and resulting current directions.

- Calculate the voltage gain (Av  VL/Vi) for the network of Fig. 3.12 if Vi  500 mV and R  1 k. (The other circuit values remain the same.)

- Calculate the voltage gain (Av  VL/Vi) for the network of Fig. 3.12 if the source has an internal resistance of 100  in series with Vi.

- Define ICBO and ICEO. How are they different? How are they related? Are they typically close in magnitude?

- Using the characteristics of Fig. 3.14:

  - Find the value of IC corresponding to VBE 750 mV and VCE 5 V.

  - Find the value of VCE and VBE corresponding to IC  3 mA and IB  30 A.

21. (a) For the common-emitter characteristics of Fig. 3.14, find the dc beta at an operating point of VCE  8 V and IC  2 mA.

(b) Find the value of  corresponding to this operating point.

(c) At VCE  8 V, find the corresponding value of ICEO.

(d) Calculate the approximate value of ICBO using the dc beta value obtained in part (a).

22. (a) Using the characteristics of Fig. 3.14a, determine ICEO at VCE  10 V.

(b) Determine dc at IB  10 A and VCE  10 V.

(c) Using the dc determined in part (b), calculate ICBO.

23. (a) Using the characteristics of Fig. 3.14a, determine dc at IB  80 A and VCE  5 V.

(b) Repeat part (a) at IB  5 A and VCE  15 V.

(c) Repeat part (a) at IB  30 A and VCE  10 V.

(d) Reviewing the results of parts (a) through (c), does the value of dc change from point to point on the characteristics? Where were the higher values found? Can you develop any general conclusions about the value of dc on a set of characteristics such as those provided in Fig. 3.14a

24. (a) Using the characteristics of Fig. 3.14a, determine ac at IB  80 A and VCE  5 V.

(b) Repeat part (a) at IB  5 A and VCE  15 V.

(c) Repeat part (a) at IB  30 A and VCE  10 V.

(d) Reviewing the results of parts (a) through (c), does the value of ac change from point to point on the characteristics? Where are the high values located? Can you develop any general conclusions about the value of ac on a set of collector characteristics?

(e) The chosen points in this exercise are the same as those employed in Problem 23. If Problem 23 was performed, compare the levels

25. Using the characteristics of Fig. 3.14a, determine dc at IB  25 A and VCE  10 V. Then calculate dc and the resulting level of IE. (Use the level of IC determined by IC  dcIB.)

26. (a) Given that dc  0.987, determine the corresponding value of dc.

(b) Given dc  120, determine the corresponding value of .

(c) Given that dc  180 and IC  2.0 mA, find IE and IB.

27. From memory, and memory only, sketch the common-emitter configuration (for npn and pnp) and insert the proper biasing arrangement with the resulting current directions for IB, IC, and IE.

28. An input voltage of 2 V rms (measured from base to ground) is applied to the circuit of Fig. 3.21. Assuming that the emitter voltage follows the base voltage exactly and that Vbe (rms)  0.1 V, calculate the circuit voltage amplification (Av  Vo /Vi) and emitter current for RE 1 k.

29. For a transistor having the characteristics of Fig. 3.14, sketch the input and output characteristics of the common-collector configuration.

30. Determine the region of operation for a transistor having the characteristics of Fig. 3.14 if ICmax  7 mA, VCEmax  17 V, and PCmax  40 mW