Nama	: Tsaniya Faridah Maulida

NIM                              : 211331031

Kelas	: 1A – Teknik Telekomunikasi

Elektronika Analog – BAB 1 No. 30 – 61

30.  a. Determine the dynamic (ac) resistance of the commercially available diode of Fig. 1.15 at a forward current of 10 mA using Eq. (1.5).

b. Determine the dynamic (ac) resistance of the diode of Fig. 1.15 at a forward current of 10 mA using Eq. (1.6).

c. Compare solutions of parts (a) and (b)

Answer

- rd=∆Vd∆Id=0,79 V-0,76 V15 mA-5 mA=0,03 V10mA=3Ω

- rd=26mVID=26 mV10mV=2,6Ω

- cukup dekat

- Calculate the dc and ac resistances for the diode of Fig. 1.15 at a forward current of 10 mA and compare their magnitudes.

Answer

ID = 10 mA, VD = 0,76 V

RDC=VdId=0,76 V10mA=76Ω

rd=∆Vd∆Id≅0,79 v-0,76 v15 mA-5mA=0,03 V10mA=3Ω

RDC≫rd

- Using Eq. (1.6), determine the ac resistance at a current of 1 mA and 15 mA for the diode of Fig. 1.29. Compare the solutions and develop a general conclusion regarding the ac resistance and increasing levels of diode current.

Answer

ID=1mA,rd=∆Vd∆Id=0,72 V-0,61 V2 mA-0mA=55Ω

ID=15mA,rd=∆Vd∆Id=0,8 V-0,78 V20 mA-10mA=2Ω

- Using Eq. (1.7), determine the ac resistance at a current of 1 mA and 15 mA for the diode of Fig. 1.19. Modify the equation as necessary for low levels of diode current. Compare to the solutions obtained in Problem 32.

Answer

ID=1mA,rd=226mVID=2(26Ω) = 52Ω vs 55Ω (#30)

ID=15mA,rd=26mVID=26mV15mA= 1,73Ω vs 2Ω (#30)

- Determine the average ac resistance for the diode of Fig. 1.19 for the region between 0.6 &0.9 V.

Answer

rav=∆Vd∆Id=0,9 V-0,6 V13,5 mA-1,2mA=24Ω

- Determine the ac resistance for the diode of Fig. 1.19 at 0.75 V and compare to the average ac resistance obtained in Problem 34.

Answer

rd=∆Vd∆Id≅0,8 V-0,7 V7 mA-3mA=0,09V4mA22,5Ω

- Find the piecewise-linear equivalent circuit for the diode of Fig. 1.19. Use a straight line segment that intersects the horizontal axis at 0.7 V and best approximates the curve for the region greater than 0.7 V.

Answer

rav=∆Vd∆Id=0,9 V-0,7 V14 mA-0mA=0,2V14mA=14,2Ω

- Repeat Problem 36 for the diode of Fig 1.2.9

Answer

Using the best approximation to the curve beyong VD=0,7 V:

rav=∆Vd∆Id=0,8 V-0,7 V25mA-0mA=0,1V25mA=4Ω

- Plot IF versus VF using linear scales for the diode of Fig. 1.36. Note that the provided graph employs a log scale for the vertical axis (log scales are covered in sections 11.2 and 11.3).

Answer

- Comment on the change in capacitance level with increase in reverse-bias potential for the diode of Fig. 1.36

Answer

Saat besar potensi bias balik meningkat maka kapasitansi turun sekitar 5pF tanpa bias. Potensi reverse-bias lebih dari 10 V kapasitansinya turun

- Does the reverse saturation current of the diode of Fig. 1.36 change significantly in magnitude for reverse-bias potentials in the range 25 to 100 V?

Answer

At VD= -25V, ID=-0,2nA and at VD=-100V,ID=-0,45nA. Although the change in IR is more than 100%, the level of IR and the resulting change is relatively small for most applications.

- For the diode of Fig. 1.36 determine the level of IR at room temperature (25°C) and the boiling point of water (100°C). Is the change significant? Does the level just about double for every 10°C increase in temperature?

Answer

Log scale: TA = 25C, IR = 0.5 nA

TA = 100C, IR = 60 nA

The change is significant. 60 nA: 0.5 nA = 120:1

Yes, at 95C IR would increase to 64 nA starting with 0.5 nA (at 25C) (and double the level every 10C).

- For the diode of Fig. 1.36 determine the maximum ac (dynamic) resistance at a forward current of 0.1, 1.5, and 20 mA. Compare levels and comment on whether the results support conclusions derived in earlier sections of this chapter.

Answer

IF = 0.1 mA: rd  700

IF = 1.5 mA: rd  70

IF = 20 mA: rd  6 , AC menurun dengan cepat seiring meningkatnya arus.

- Using the characteristics of Fig. 1.36, determine the maximum power dissipation levels for the diode at room temperature (25°C) and 100°C. Assuming that VF remains fixed at 0.7 V, how has the maximum level of IF changed between the two temperature levels?

Answer

T=25°C:PMAX=500mW

T=100°C:PMAX=260mW

PMAX=VFIF

IF=PMAXVF=500mW0,7V=714,29mA

IF=PMAXVF=260mW0,7V=371,43mA

714,29mA:371,43mA = 1,92:1 = 2:1

- Using the characteristics of Fig. 1.36, determine the temperature at which the diode current will be 50% of its value at room temperature (25°C).

Answer

Using the bottom right graph of Fig. 1.37:

IF= 500 mA @ T = 25C

AtIF = 250 mA, T  104C

- (a) Referring to Fig. 1.37, determine the transition capacitance at reverse-bias potentials of 25 and 10 V. What is the ratio of the change in capacitance to the change in voltage?

(b) Repeat part (a) for reverse-bias potentials of 10 and 1 V. Determine the ratio of the change in capacitance to the change in voltage.

(c) How do the ratios determined in parts (a) and (b) compare? What does it tell you about which range may have more areas of practical application?

Answer

- VR= -25 V:Cr≅0,75pF

VR= -10 V:Cr≅1,25pF

∆CT∆VR=1,25 pF-0,75pF10V-25V=0,5pF15V=0,033pF/V

- VR= -10 V:Cr≅1,25 pF

VR= -1 V:Cr≅3 pF

∆CT∆VR=1,25 pF-3pF10V-1V=1,75pF9V=0,194pF/V

- 0,194pF/V : 0,033 pF/V = 5,88:1 = 6:1 Increased sensitivity near Vd=0V

- Referring to Fig. 1.37, determine the diffusion capacitance at 0 and 0.25 V.

From Fig. 1.33

Answer

VD = 0 V, CD = 3.3 pF

VD = 0.25 V, CD = 9 pF

- Describe in your own words how diffusion and transition capacitances differ.

Answer

Kapasitansi difusi ditentukan oleh laju injeksi muatan ke daerah tepat diluar batas penipisan bias majunya. Kapasitansi transisi disebabkan adanya penipisan daerah seperti elektrik di daerah bias mundur.

- Determine the reactance offered by a diode described by the characteristics of Fig. 1.37 at a forward potential of 0.2 V and a reverse potential of 20 V if the applied frequency is 6 MHz.

Answer

VD = 0,2 V, CD = 7,3 pF

XC=12πfC=12π(6MHz)(7,3pF)=3,64kΩ

VD = -20 V, Cr = 0,9 pF

XC=12πfC=12π(6MHz)(0,9pF)=29,47kΩ

- Sketch the waveform for i of the network of Fig. 1.61 if Tt 2ts and the total reverse recovery time is 9 ns.

Answer

- The following characteristics are specified for a particular Zener diode: VZ = 29 V, VR = 16.8 V, IZT = 10 mA, IR = 20 A, and IZM = 40 mA. Sketch the characteristic curve in the manner displayed in Fig. 1.50.

Answer

- At what temperature will the 10-V Zener diode of Fig. 1.50 have a nominal voltage of 10.75 V? (Hint: Note the data in Table 1.4.)

Answer

TC=+0,072%= ∆VzVzT1-T0x100%

0,072 = 0,75V10VT1-25x100%

0,072=7,5T1-25

T1-25°=7,50,072 = 104,17° = 129,17°

- Determine the temperature coefficient of a 5-V Zener diode (rated 25°C value) if the nominal voltage drops to 4.8 V at a temperature of 100°C.

Answer

TC=∆VzVzT1-T0x100%

= (5V-4,8V)5V (100°-25°)X100%=0,053%/°C

- Using the curves of Fig. 1.51a, what level of temperature coefficient would you expect for a 20-V diode? Repeat for a 5-V diode. Assume a linear scale between nominal voltage levels and a current level of 0.1 mA.

20V-6,8V24V-6,8VX100%=77%

Zener 20 V adalah 77% dari jarak antara 6,8 V dan 24 V yang diukur dari karakteristik 6,8 V.

At Iz=0,1 mA, Tc ≅0,06%/°C

5V-3,6V6,8-3,6VX100%=44%

Zener 5 V adalah 44% dari jarak antara 3,6 V dan 6,8 V yang diukur dari karakteristik 3,6 V.

At Iz=0,1 mA, Tc ≅-0,025%/°C

- Determine the dynamic impedance for the 24-V diode at IZ = 10 mA for Fig. 1.51b. Note that it is a log scale.

Answer

- Compare the levels of dynamic impedance for the 24-V diode of Fig. 1.51b at current levels of 0.2, 1, and 10 mA. How do the results relate to the shape of the characteristics in this region

Answer

24 V Zener:

0.2 mA:  400

1 mA:  95

10 mA:  13

- Referring to Fig. 1.55e, what would appear to be an appropriate value of VT for this device? How does it compare to the value of VT for silicon and germanium?

Answer

VK 2.0 V, which is considerably higher than germanium ( 0.3 V) or silicon ( 0.7 V). For germanium it is a 6.7:1 ratio, and for silicon a 2.86:1 ratio.

- Using the information provided in Fig. 1.55, determine the forward voltage across the diode if the relative luminous intensity is 1.5.

Answer

Fig. 1.53 (f) IF  13 mA

Fig. 1.53 (e) VF  2.3 V

- (a) What is the percent increase in relative efficiency of the device of Fig. 1.55 if the peak current is increased from 5 to 10 mA?

(b) Repeat part (a) for 30 to 35 mA (the same increase in current).

(c) Compare the percent increase from parts (a) and (b). At what point on the curve would you say there is little gained by further increasing the peak current?

Answer

- Relative efficiency @ 5 mA  0.82 ; @ 10 mA  1.02

1,02-0,820,82x100%=24,4% increase ratio : 1,020,82=1,24

- Relative efficiency @ 30 mA  1.38 ; @ 35 mA  1.42

1,42-1,381,38x100%=1,9% increase ratio : 1,421,38=1,03

- Arus yang lebih besar sekitar 30 mA, kenaikan persennya lebih kecil dibanding dengan kenaikan arus yang nilainya lebih kecil.

- (a) Referring to Fig. 1.55h, determine the maximum tolerable peak current if the period of the pulse duration is 1 ms, the frequency is 300 Hz, and the maximum tolerable dc current is 20 mA. (b) Repeat part (a) for a frequency of 100 Hz.

Answer

- (a) If the luminous intensity at 0° angular displacement is 3.0 mcd for the device of Fig. 1.55, at what angle will it be 0.75 mcd?

(b) At what angle does the loss of luminous intensity drop below the 50% level?

Answer

a. 0,753,0=0,25 ≅75%

b. 0,5 = 40°

- Sketch the current derating curve for the average forward current of the high-efficiency red LED of Fig. 1.55 as determined by temperature. (Note the absolute maximum ratings.)

0,2 mA℃=20mAX

x=20mA0,2mA/℃=100℃