SOAL ANALOG BAB 8

- For the network of Fig. 8.64:

- Determine Zi and Zo

- re : IB = Vcc-VBERB = 12 V - 0.7 V220K Ω = 51.36 µA

IE = ( ß + 1 ) IB = (60 + 1)(51.36 µA) = 3.13 mA

re = 26 mVIE = 26 mV3.31 mA = 8.31 Ω

Zi = RB || ßre = 220K Ω || (60)(8.31 Ω) = 220K Ω || 498.6 Ω = 497.47 Ω

ro ≥ 10Rc * Zo = Rc = 2.2K Ω

- Find Av and Zo

- Av = - Rcre = - 2.2K Ω8.31 Ω = -264.74

Zo =

- Repeat part a with ro 20K Ω

- Zi = 497.47 Ω

Zo = ro || Rc = 20K Ω || 2.2K Ω = 1.98K Ω

- Repeat part b with ro 20K  Ω

- Av = -Rc ||rore = -1.98K Ω8.31 Ω = -238.27

- For the network of Fig 8.65 determine Vcc for a voltage gain of Av = -200

- Av = - Rcre re = - RcAv = - 4.7K Ω-200 = 23.5 Ω

re =  26 mVIE IE = 26 mVre = 26 mV23.5 Ω = 1.106 Ma

IB = IEß+1 = 1.106 mA91 = 12.15 µA

IB = Vcc-VBERB

Vcc = IBRB + VBE = (12.15 µA)(1M Ω) + 0.7 V = 12.15 V + 0.7 V = 12.85 V

- For the network of Fig 8.66 :

- Calculate IB, IC, and re

IB = Vcc-VBERB = 10 V-0.7 V390K Ω = 23.84 µA

IE = (ß+1)IB=(101)(23.84 µA) = 2.40 mA

re =  26 mVIE = 26 mV1.89 mA = 13.76 Ω

IC = ßIB = (100)(23.84 µA) = 2.384 mA

- Determine Zi and Zo

Zi = RB || ßre = 390K Ω || (100)(13.76 Ω) = 1.38K Ω

Ro ≥ 10Rc *Zo = Rc = 5.6K Ω

- Calculate Av and Ai

Av = - Rcre = -5.6K Ω13.76 Ω = -406.98

- Determine the effect of ro – 30K Ω and Av and Ai

Av = - Rc ||r0re = - 5.6K Ω||(30K Ω)13.76 Ω = - 4.72K Ω13.76 Ω = -343.03

- For the network of Fig 8.67 :

- Determine re

VB = R2 VCCR1+R2 = 4.7K Ω (16 V)39K Ω+4.7K Ω = 1.721 V

VE = VB-VBE = 1.721 V – 0.7 V = 1.021 V

IE = VERE = 1.021 V1,2K Ω = 0.8507 mA

re = 26 mVIE = 26 mV0.8507 mA = 30.56 Ω

- Calculate Zi and Zo

Zi = R1 || R2 || ßre

= 4.7K Ω ||39K Ω || (100)930.56 Ω) = 1.77K Ω

ro ≥ 10Rc *Zo = Rc = 3.9K Ω

- Find Av and Ai

Av = - Rcre = - 3.9K Ω30.56 Ω = -127.6

- Repeat parts (b) and (c) with ro = 25K Ω

Zi = 1.77K

Zo = Rc || ro = 3.9K Ω || 25K Ω = 3.37K Ω

Av = - (Rc ||r0)re = - 3.9K Ω||(25K Ω)30.56 Ω = - 3.37K Ω30.56 Ω = -110.28

- Determine Vcc for the network of Fig. 8.68 if Av = -160 and ro = 100K Ω

Av = = - Rcre = re = = - RcAv = = - 3.3K Ω-160 = 20.625 Ω

re =  26 mVIE = IE = = - 26 mVre =  26 mV20.625 Ω = 1.261 mA

IE =  VERE = VE =IERE = (1.261 mA)(1K Ω) = 1.261 V

VB = VBE + VE = 0.7 V + 1.261 V = 1.961 V

VB = 5.6K Ω VCC5.6K Ω+82K Ω = 1.961 V

5.6K Ω VCC = (1.961 V)(87.6K Ω)

VCC = 30.68 V

- For the network of fig 8.69

- Determine re

Rth = 56K Ω || 220K Ω = 44.64K Ω

Eth = 56K Ω (20 V)220K Ω+56K Ω = 4.058 V

IB = Eth-VBERth+ß+1RE = 4.058 V-0.7 V44.64K Ω+(181)(2.2K Ω) = 7.58 µA

IE = (ß + 1)IB = (181)(7.58 µA) = 1.372 mA

re = 26 mVIE = 26 mV1.372 mA = 18.95 Ω

- Calculate Vb and Vc

VE = IERE=(1.372 mA)(2.2K Ω) = 3.02 V

VB = VE + VBE = 3.02 V + 0.7 V = 3.72 V

VC = VCC – ICRC = 20V – ßIBRC = 20 V – (180)(7.58 µA)(4.7K Ω) = 13.59 V

- Determine Zi and Av = V0/Vi

Zi = R1 || R2 || ßre

= 56K Ω || 220K Ω || (180)(7.58 µA) = 44.64K Ω || 3.14K Ω = 3.17K Ω

ro < 10Rc

33.33K Ω < 10(4.7K Ω) = 47K Ω

Av =  - Rc ||r0re = - 6.8K Ω||(50K Ω)18.95 Ω  = -298.15

- For the network of Fig 8.70

- Determine re

- Find Zi and Zo

- Calculate Av and Ai

- Repeat parts b and c with ro = 20K Ω

8.  For the network of Fig. 8.71, determine RE and RB if Av  10 and re  3.8 . Assume that

Zb  RE.

Jawabannya

9.  Repeat Problem 7 with RE bypassed. Compare results.

Jawabannya

10. . For the network of Fig. 8.72:

(a) Determine re.

(b) Find Zi and Av.

(c) Calculate Ai.

Jawabannya

11. For the network of Fig. 8.73:

(a) Determine re and re.

(b) Find Zi and Zo.

(c) Calculate Av and A

Jawabanya

12. For the network of Fig. 8.74:

(a) Determine Zi and Zo.

(b) Find Av.

(c) Calculate Vo if Vi  1 mV.

Jawabannya

13. . For the network of Fig. 8.75:

(a) Calculate IB and IC.

(b) Determine re.

(c) Determine Zi and Zo.

(d) Find Av and Ai.

Jawabannya

14. For the common-base configuration of Fig. 8.76:

(a) Determine re.

(b) Find Zi and Zo.

(c) Calculate Av and Ai.

Jawabannya

15. . For the network of Fig. 8.77, determine Av and Ai.

Jawabannya

For the network of Fig. 8.64:

(a) Determine Zi and Zo.

(b) Find Av and Ai

(c) Repeat part (a) with ro = 20 kΩ.

(d) Repeat part (b) with ro = 20 kΩ.

For the network of Fig. 8.65, determine VCC for a voltage gain of Av = - 200.

For the network of Fig. 8.66:

(a) Calculate IB, IC, re.

(b) Determine Zi and Zo.

(c) Calculate Av and Ai

(d) Determine the effect of ro = 30 kΩ on Av and Ai.

For the network of Fig. 8.67:

(a) Determine re.

(b) Calculate Zi and Zo.

(c) Find Av and Ai.

(d) Repeat parts (b) and (c) with ro = 25 kΩ.

Determine VCC for the network of Fig. 8.68 if Av= -160 and ro = 100 kΩ.

For the network of Fig. 8.69:

(a) Determine re.

(b) Calculate VB and VC.

(c) Determine Zi and Av=  VoVi

For the network of Fig. 870 :

(a) Determine re.

(b) Find Zi and Zo.

(c) Calculate Av and Ai.

(d) Repeat parts (b) and (c) with ro = 20 kΩ.

For the network of Fig. 8.71, determine RE and RB if Av= - 10 and re= 3.8Ω. Assume that Zb= βRE.

- Repeat Problem 7 with RE bypassed. Compare results.

For the network of Fig. 872 :

(a) Determine re.

(b) Find Zi and Av.

(c) Calculate Ai.

For the network of Fig. 873 :

(a) Determine re and βrE.

(b) Find Zi and Zo.

(c) Calculate Av and Ai.

For the network of Fig. 874 :

(a) Determine Zi and Zo.

(b) Find Av.

(c) Calculate Vo if Vi = 1 mV.

For the network of Fig. 875 :

(a) Calculate IB and IC.

(b) Determine re.

(c) Determine Zi and Zo.

(d) Find Av and Ai.

For the common-base configuration of Fig. 8.76:

- Determine re.

- Find Zi and Zo.

- Calculate Av and Ai.

For the network of Fig. 8.77, determine Av and Ai.

For the collector FB configuration of Fig. 8.78:

- Determine re

IB = VCC- VBERF+ βRC= 12 V-0.7 V220kΩ+120(3.9kΩ)=16.42µA

IE=β+1IB=120+116.42µA=1.987mA

re= 26 mVIE=26 mmV1.987mA=13.08 Ω

- Find Zi and Zo

Zi= βre||RFAv

Av= -RCre=-3.9 kΩ13.08Ω= -298

Zi= (120)(13.08Ω) || 220kΩ298 = 1.5696 kΩ||738Ω=501.98 Ω

Zo= RC|| RF=3.9 kΩ || 220kΩ = 3.83 kΩ

- Calculate Av and Ai

From above, Av= -298

Given re. = 10Ω, β = 200, Av = - 160, and Ai= 19 for the network of Fig. 8.79, determine RC, RF, and VCC.

Av= -RCre = - 160

RC= 160(re) = 160(10Ω) = 1.6 kΩ

Ai= βRFRF+ βRC=19→19= 200RFRF+200(1.6 kΩ) = RF= 3800RC181=3800(1.6 kΩ)181=33.59 kΩ

IB= VCC-VBERF+βRC= IBRF+βRC= VCC-VBE and VCC= VBE+ IBRF+βRC

with IE= 26 mVre= 26 mV10Ω=2.6 mA→ IB= IEβ+1= 2.6mA200+1=12.94 µA

∴ VCC = VBE+IBRF+βRC=0.7 V+12.94 µA33.59 kΩ+2001.6 kΩ=5.28 V

- For the network of Fig. 8.30:

- Derive the approximate equation for Av.

Av : Vi= Ibβre+ (β+1) IbRE

Io+ I'= IC= βIb, but Ii= I'+ Ib and I'= Ii-Ib

Substituting, Io+Ii- Ib=βIb and Io=β+1Ib-Ii

Assuming β+1Ib≫Ii → Io≅ β+1Ib and  Vo= - Io RC= -β+1IbRC

Therefore, VoVi=-β+1IbRCIbβRE+β+1IbRE and Av =VoVi≅ - RCre+RE= -RCRE

(b) Derive the approximate equation for  Ai.

(c) Derive the approximate equations for Zi and Zo.

Vi≅βIb(re+RE) for re≪RE → Vi≅βIbRE → Now Ii+ I'+ Ib= Vi-VoRF + Ib

Since Vo≫Vi → Ii=-VoRF+Ib or Ib= Ii+VoRF and Vi=βREIi+βVoRFRE but Vo= AvVi

and Vi= βREIi+βAvViRERF  or Vi-AvβREViRF= βREIi

(d) Given RC = 2.2kΩ, RF = 120 kΩ, RE = 1.2 kΩ, β =  90, and VCC = 10 V, calculate the magnitudes of Av,  Ai, Zi and Zo using the equations of parts (a) through (c).

Av ≅ -RCRE = - 2.2kΩ1.2kΩ= -1.83

Zi≅βRERFβREAV+RF=901.2kΩ120kΩ901.2kΩ1.83+120kΩ= 40.8 kΩ

Zo≅ RC|| RF=2.2kΩ|| 120kΩ = 2.16 kΩ

For the network of Fig. 8.80:

- Determine Zi and Zo.

IB=VCC-VBERF+βRC=9 V-0.7 V (39 kΩ+22kΩ)(80)(1.8 kΩ)= 8.3 v61 kΩ+144 kΩ=8.3 V205 kΩ=40.49 µA

IE=β+1IB=80+140.49 µA=3.28 mA

re= 26 mVIE=26 mV3.28 mA=7.93 Ω

Zi= RFi|| βre= 39 kΩ || (80)(7.93 Ω)= 39 kΩ || 634.4 Ω = 0.62 kΩ

Zo= RC|| RF2=1.8 kΩ|| 22 kΩ = 1.66 kΩ

- Find Av and Ai.

- (a) Given β = 120, re = 4.5Ω and ro= 40kΩ, sketch the approximate hybrid equivalent circuit.

hfe= β = 120

hie ≅ βre = (120)( 4.5Ω) = 540 Ω

hoe= 1ro= 140 kΩ=25 µS

(b) Given hie= 1 kΩ, hre=2×10-4, hfe=90, and hoe=20µS, skecth the re model.

re≅ hieβ= 1kΩ90=11.11 Ω

β= hfe=90

ro= 1hoe= 120 µS=50 kΩ

- For the network of Problem 1:

- Determine re=8.31 Ω

- hfe= β = 60

hie = βre = (60)( 8.31 Ω) = 498.6 Ω

- Find Zi and Zo. using the hybrid parameters.

Zi= RB|| hie = 220 kΩ || 498.6 Ω = 497.47 Ω

Zo= RC=2.2 kΩ

- Calculate Av and Ai using the hybrid parameters.

Av= -hfeRChie= -(60)(2.2kΩ)498.6 Ω = -264.74

Ai≅hfe= 60

- Determine Zi and Zo If hoe= 50 µS.

Zi=497.47 Ω (the same)

Zo= ro|| RC,  ro = 125 µS=40 kΩ → 40 kΩ || 2.2 kΩ = 2.09 kΩ

- Determine Av and Ai If hoe= 50 µS.

Av= -hfe(ro||RC)hie= -(60)(2.085 kΩ)498.6 Ω = -250.90

Ai= -AvZiRC= --250.90497.47Ω2.2 kΩ=56.73

(g) Compare the solutions above with those of Problem 1. (Note: The solutions are available in Appendix E if Problem 1 was not performed.)

For the network of Fig. 8.81 :

- Determine Zi and Zo.

68 k  12 k = 10.2 k

Zi = 10.2 k  hie = 10.2 k  2.75 k = 2.166 k

ZO = RC  ro  = 2.2 k  40 k = 2.085 k

- Calculate Av and Ai.

Av= -hfeR'Chie →R'C= RC  || ro=2.085 kΩ

= -(180)(2.085 kΩ)2.75  kΩ = -136.5

Ai= IOIi= IOI'i × I'iIi = 135.13

- Determine reand βre to hie.

For the common-base network of Fig. 8.82:

- Deterime Zi = RE  hib  = 1.2 k  9.45  = 9.38

Zo = RC  1hib= 2.7 k  11×10-6A/V = 2.7 k  1 M ≅ 2.7 k

- Av= -hfb(RC  1hab)hib= -(-0.992)(≅2.7 k)9.45 =284.43

Ai≅-1

- Determine α, β, reand ro.

α = -hfb= --0.992=0.992

β = α1-α= 0.9921-0.992=124

re=hib=9.45 Ω

ro= 1hab= 11µA/V=1 MΩ

- Repeat parts (a) and (b) of Problem 22 with hre=2×10-4 and compare results.

For the network of Fig. 8.83, determine:

- Zi'= hie= hfehreRL1+hoeRL=0.86kΩ-1401.5x10-42.2kΩ1+25 µS2.2kΩ=0.86kΩ-43.79Ω=816.21 Ω

Zi=RB||Zi'=470 kΩ ||816.21 Ω = 814.8 Ω

- Av = -hfeRLhie+hiehoe-hfehreRL=-1402.2kΩ0.86kΩ+0.86kΩ25 µS-(140)1.5x10-4)2.2kΩ= -357.68

- Ai = IOIi=hfe1+hoeRL= 1401+25 µS2.2kΩ=132.70

- Zo= 1hoe-(hfehre/(hie+Rs)=125 µS-((140)1.5x10-4)(0.86kΩ+1kΩ))= 113.71µS=72.9kΩ

For the common-base amplifier of Fig. 8.84, determine:

- Zi'=hib- hfbhrbRL1+hobRL=9.45kΩ--0.9971x10-42.2kΩ1+0.5 µAV2.2kΩ=9.67Ω

Zi=1.2 kΩ||Zi'= 1.2 kΩ ||9.67 Ω = 9.59 Ω

- Ai'=hfb1+hobRL=0.9971+0.5 µAV2.2kΩ=-0.996

Ai=IoIi'.Ii'Ii=(-0.996)1.2 kΩ1.2 kΩ+9.67 kΩ = - 0.998

- Av=-hfLRLhib+hibhab-hfbhfbRL=--0.9972.2kΩ9.45Ω+9.45Ω0.5µA/V-(0,997)1x10-4)2.2kΩ= 226.21

- ZO'=1hab-hfbhrb/hib=10.5µA/V-((-0,997)1x10-4)/9.45 Ω=90.5kΩ

Zo = 2.2k|| ZO' = 2.15kΩ

Given the network of Fig. 8.85:

- Determine if the system is operating properly based on the voltage-divider bias levels and expected waveforms for Vo and VE.

- Determine the reason for the dc levels obtained and why the waveform for Vo was obtained.

- Using PSpice Windows, determine the gain for the network of Fig. 8.6. Use Probe to display the input and output waveforms.

- Using PSpice Windows, determine the gain for the network of Fig. 8.13. Use Probe to display the input and output waveforms.

- Using PSpice Windows, determine the gain for the network of Fig. 8.25. Use Probe to display the input and output waveforms.